Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	INFORMATION THEORY AND CODING			      	     Time : 3 hours

Subject Code:	EC290							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define entropy.

2.	Define uncertainty.

3.	What is companding?

4.	What is the other name for 1 bit modulator?

5.	Define dmin.

6.	What is systematic code word?

7.	Expand JPEG.

8.	Why image compression techniques are required?

9.	What is predictive coding?

10.	What is the need for video coding?

PART – B  (5 x 3 = 15 MARKS)

11.	An analog signal is bandlimited to B Hz, sampled at the Nyquist rate, and the samples are quantized into 4 levels. The quantization levels Q1, Q2, Q3 and Q4 (messages) are assumed independent and occur with probabilities p1=p4=1/8 and p2=p3=3/8. Find the information rate of the source.

12.	Write short notes on Vocoders.

13.	State the properties of cyclic codes.

14.	Write short notes on graphics interchange format.

15.	Write short notes on video compression.

PART – C  (5 x 15 = 75 MARKS)

16.	State and prove the properties of entropy.

(OR)

17.	Consider a discrete memory less source with seven possible symbols xi, i = 1,2,…..7 and the corresponding probabilities P(x1)=0.37, P(x2) =0.33, P(x3) = 0.16, P(x4)=0.07, P(x5)=0.04, P(x6)=0.02 and P(x7)=0.01.  Determine the code word, Lmin and efficiency of the code using Huffman coding schemes.

18.	With a suitable block diagram and wave forms comprehend on Delta modulation. Enlist its advantages and disadvantages too.

(OR)

19.	With a suitable block diagram and wave forms comprehend on DPCM.

[P.T.O]

20.	The parity check matrix of a linear (7, 4) block code is given as follows. Show how the data words 	a.	0011	b.	0100 	c.	0101 are coded.

H=101110011010100111001

(OR)

21.	Consider the generator of a (7, 4) cyclic code by generator polynomial g(D)=1+D+D3.  Calculate the code word for the message sequence 1001 and construct the systematic generator matrix G. Draw the diagram of the encoder.

22.	Encode the sequence 1321 whose cumulative density function is Fx(k)=0, k<=0, Fx(1)=0.8, Fx(2)=0.82, Fx(3)=1, Fx(k)=1,k.>3. Generate the tag and decipher it using Arithmetic coding.

(OR)

23.	Comprehend on Static and dynamic Huffman Coding.

24.	Write short notes on:

a.	Linear Predictive Codes														    (7)

b.	Code Excited Linear Predictive Codes											    (8)

(OR)

25.	Explain H.261 and MPEG video standards.