Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS			      	     Time : 3 hours

Subject Code:	EE203/EE295									       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An admittance function is given by Y(s) = 1/(s+2). Draw the pole-zero plot.

2.	List any one condition for reciprocity.

3.	Define Resonant frequency.

4.	Give the significance of Bode-plot.

5.	Define Impedance.

6.	What is cut-set matrix?

7.	Define propagation constant in two port network.

8.	List any two properties of a filter.

9.	Check whether the polynomial s8+s7+s6-2s5+4s3-s2+s+1 is Hurwitz or not.

10.	Give any one property of a positive real function.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the final value of the current in a RL circuit following step response using final value theorem.

12.	Assume that the output of a circuit depends on frequency according to

V= (Aω)/ (sqrt(B+Cω2)

Find the break frequency.

13. 	List the two properties of Incidence matrix.

14.  	Discuss the disadvantages of constant K filters.

15. 	State whether the following function is driving point immittances of LC network or not

Z(s)= (10(s2+4)(s2+6) / ((s2+1)(s2+9))

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Check whether the polynomial P(s) = s4+s3+2s2+2s+3 is stable or not. Comment 	on your 	result. 																		  (10)

b.	Explain, with reasons, why the following expression for the driving point impedance Z(s) 	is not suitable for representing a passive network.								   (5)

Z(s) = (s4–s3+2s2)/(s+5)

(OR)

17.	a.	Determine the poles of the circuit shown in figure.								  (5)

[P.T.O]

b.	A current transfer function is given by

I(s) = 5s/ ((s+2) (s2+2s+2)). Obtain its time domain response.						(10)

18.	Draw the pole zero plot for given network function.

v(s)=4s(s+2)s+1s+3

(OR)

19.	a.	Give the properties of Herwitz polynomial.										(10)

b.	Derive h – parameter in terms of z parameter.									  (5)

20.	a.	Explain the principle of duality with an illustration.								   (8)

b.	For the figure, draw its graph. Select a suitable tree and obtain its tie-set matrix.  Write 	down the KVL equations from the tie-set matrix.									   (7)

(OR)

21.	a.	For the graph shown in figure, develop the fundamental cut-set matrix.				  (10)

b.	Draw the dual of the network shown in figure.									    (5)

22. 	For the network shown in figure , find Z-parameters?

(OR)

23.	a.	Design a constant K-low pass filter having cut-off frequency 2.5 kHz and design resistance 	Ro=700 ohms. Also find the frequency at which this filter produces attenuation of 19.1 dB. 	Find its characteristic impedances and phase constant at pass band and stop or attenuation 	band.																		    (8)

b.	Design a second order Butterworth low pass filter with cut-off frequency of 1 kHz.	    (7)

24.	A function is given by

Z(s)= (s3+5s2+9s+3)/(s3+4s2+7s+9). Find the positive realness of the function.

(OR)

25.	a.	The driving point impedance of a one port reactive network is given by

Z(s)= (4(s(s2+4))) / ((s2+1)(s2+16))

Obtain the foster forms of LC network realization. 								  (10)

b.	The driving point impedance of a network is given by

Z(s)= (6s3+2s)/ (9s4+4s2+(1/6))

Find the first form of Cauer network.											    (5)