Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRIC CIRCUITS								      	     Time : 3 hours

Subject Code:	EE245							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give any one example for active element in a electric circuit.

2.	In a parallel network, if any one of the resistance is removed, the effective resistance of the parallel circuit will ____________.

3.	The value of form factor of sinusoidal waveform is ____________.

4.	Draw the power triangle of series RL circuit.

5.	Mesh analysis is applicable only for ____________ circuits.

6.	Give the expression for the number of mesh currents in terms of number of nodes and number of branches.

7.	Super position theorem is applicable for non-linear circuits also. Say true or false.

8.	What is the condition for maximum power transfer between source and load in a dc circuit?

9.	What is the condition for series resonance electrical circuit?

10.	Define Quality factor.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the classifications of circuit elements?

12.	A sinusoidal voltage is applied in series to a capacitor 0.01μF. The frequency of the sine wave is 2 kHz. Determine the capacitive reactance of the circuit.

13.	What are the properties of super-mesh?

14.	Transform the given voltage source in figure into current Source.

15.	What are the properties of resonance of RLC series circuit?

PART – C  (5 x 15 = 75 MARKS)

16.	Find the branch current I1, I2, and I3 using Kirchoff’s laws.

[P.T.O]

(OR)

17.	Find the equivalent resistance, REQ for the following resistor combination circuit.

18.	Obtain the expression for the average value and rms value of alternating sinusoidal wave v=Vm sin ωt.

(OR)

19.	Draw the RLC series circuit supplied with ac sinusoidal voltage and draw the voltage and current phasor diagram with the relevant expressions.

20.	Find the Nodal Voltages and also find the current flowing through various resistances.

(OR)

21.	Use mesh analysis to find the current I0 in the circuit.

22.	Find the current in 6 Ω resistor using the principle of superposition for the circuit of Fig.

(OR)

23.	Determine current flowing through 5 Ω resistance using Thevenin’s Theorem. Also find the Voltage across the resistance.

24.	A series RLC circuit has R=2Ω, L=2mH, C=10μF. Calculate (a) the resonant frequency

(b) Q factor of the circuit, (c) the Bandwidth, and (d) the half power frequency f1 and f2.

(OR)

25.	Determine the value of R for resonance in the circuit shown in figure.