Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	NETWORK THEORY								      	     Time : 3 hours

Subject Code:	EE246							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a three phase unbalanced star connected system, the vector sum of the currents in the three lines is ___________.

2.	What is the relation between phase voltage and line voltage in the star connected load?

3.	Find the coefficient of coupling of two coils having L1 = 2 H, L2 = 8 H, M = 3 H?

4.	Write two applications of tuned circuit.

5.	The time constant of a series RL circuit is ___________.

6.	Define forced response.

7.	A low pass filter is one which passes ___________ frequencies.

8.	One decibel is equal to ___________.

9.	A transfer junction is given by Z(S)=(S+2)(S+3)(S+5) .  Draw its pole-zero plot.

10.	In the first Foster form, the presence of first element capacitor Co indicates ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Derive an expression for phase angle ϕ, when the power is measured by two wattmeter method.

12.	Obtain an expression for equivalent inductance, when the 2 coils are coupled in series.

13.	A series RL circuit with R = 30Ω and L = 15H has a constant voltage V = 60V applied at t = 0.  	Determine the current I, the voltage across resistor, and the voltage across inductor.

14.	Define Neper frequency and real frequency.

15.	List out any three properties of Hurwitz polynomial.

PART – C  (5 x 15 = 75 MARKS)

16.	A three phase system with an effective voltage of 400V, has a balanced load with 	impedances 3+j4Ω. Calculate the:

a. 	phase currents 		b.	line currents.

(OR)

17.	A balanced delta connected load of (2+j3) ohm per phase is connected to a balanced three-phase 440V supply. The phase currents is 10A. Find the (a) total active power  (b) reactive power  (c) apparent power in the circuit.

18.	Derive an expression of Single Tuned circuit to find the resonant frequency, output voltage across variable capacitor and amplification factor.

(OR)

19.	Two inductively coupled coils have self-inductances L1 = 50 mH and L2 = 200 mH. If the coefficient of coupling is 0.5, find the value of mutual inductance between the coils. What is the maximum possible mutual inductance?

[P.T.O]

20.	Derive an expression of Transient analysis of series RC circuits for DC input.

(OR)

21.	List out the necessary condition for a driving point function and transfer function.

22.	Design a low pass filter (both π and T-sections) having a cut-off frequency of 2 kHz to operate with a terminated load resistance of 500Ω.

(OR)

23.	Design and draw a high pass filter having a cut off frequency of 1kHz with a load resistance of 600Ω.

24.	The driving point impedance of a one port reactive network is given by,

Z(s)=5S2+4S2+25sS2+16.

Obtain the first and second Foster networks.

(OR)

25.	Find the first and second Cauer forms of the given function.

Zs=s+1s+3ss+2.