Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	COMPUTER COMMUNICATION					      	     Time : 3 hours

Subject Code:	EE269											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example of circuit switched network.

2.	Define Protocol.

3.	Which protocol combines the features of Ethernet and token ring?

4.	Mention the length of the Ethernet cable 10BASE5 standard.

5.	Define bit stuffing.

6.	Give an example of Asynchronous protocol.

7.	List the OSI layers used in routers.

8.	Write the size of IP address.

9.	Mention the data rate used by BISDN.

10.	What is the size of cell in ATM network?

PART – B  (5 x 3 = 15 MARKS)

11.	Write notes on ARPANET.

12.	List the advantages of optical fiber cable.

13.	Draw the BSC control frame format.

14.	Discuss the types of bridges.

15.	List the functions of channels available in ISDN.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the structure of OSI model and explain the functions of each layer.

(OR)

17.	Narrate the different types of switching in communication networks. Explain in detail about packet switching.

18.	Discuss the different types of topology used in computer networks.

(OR)

19.	a.	Describe how data are communicated through the channel accessing method token ring. (8)

b.	Draw and explain the structure of coaxial cable.									    (7)

20.	a.	Narrate the different configuration available in HDLC.							    (7)

b.	Discuss the functions of X.25 layers in relation to OSI layers.						    (8)

(OR)

21.	Describe the following error control methods used in data communication

a.	Stop and wait ARQ

b.	Go-back-n ARQ

[P.T.O]

22.	a.	Determine the class, netid, hostid of the given IP addresses

i.	44.23.145.90

ii.	227.34.78.7

iii.   129.6.8.4

iv.   198.76.9.23																    (8)

b.	Illustrate  the principle of congestion control algorithm with an example.			    (7)

(OR)

23.	Explain the functions of each layer in TCP/IP protocol suit with necessary diagram.

24.	a.	Discuss the services provided by ISDN. 											    (8)

b.	Draw and explain the structure of ISDN layers.									    (7)

(OR)

25.	a.	With the architecture, discuss the features of ATM network.					      	  (10)

b.	Write notes on VSAT network.													    (5)