Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	POWER SYSTEM CONTROL						      	     Time : 3 hours

Subject Code:	EE275							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Electric Power System.

2.	Define spinning reserve.

3.	Define Load Frequency Control.

4.	Clarify the concept of control area.

5.	List the various functions of an Excitation system.

6.	Mention the benefits of power factor correction.

7.	Justify the need for SCADA in Power Systems.

8.	Define state estimation of power system.

9.	Write the coordination equation taking the effect of transmission losses.

10.	Define base point and participation factor.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the reasons for maintaining system frequency within a stringent range.

12.	Mention the cause and effect of steady state instability.

13.	Write short notes on synchronous Condensers.

14.	List the functions of Energy Control Center.

15.	What is meant by incremental cost curve and incremental operating cost?  Explain with suitable characteristics curve.

PART – C  (5 x 15 = 75 MARKS)

16.	Classify the power system load models and mention their characteristics.

(OR)

17.	Draw the functional diagram of real power control mechanism of a generator.  Explain about the cross coupling between p–f and q–v controls.

18.	With a functional block diagram, analyze the LFC of an isolated power system under steady state condition.

(OR)

19.	Draw the transfer function block diagram for a two area system provided with governor control and obtain the steady state frequency error following a step load change in both the areas.

20.	Draw the functional block diagram of AVR loop. What is the need for stability compensation?

(OR)

21.	Discuss in detail the Static VAR compensator.

22.	Explain the basic functions of SCADA in power systems.

(OR)

23.	Explain the four operating states of a power system in the security perspective.

[P.T.O]

24.	Explain in detail the lambda iteration method for solving economic scheduling problem, without transmission loss.

(OR)

25.	The incremental fuel costs for two generating units 1 and 2 of a power plant are given by the following equations.

∂F1∂P1=0.15P1+27.5 RsMW−hr

∂F2∂P2=0.26P2+19.5 RsMW−hr

Where F is fuel cost in `Rs/hr. and P is power output in MW. Determine (a) the economic loading of the two units when the total load supplied by the power plants is 100 MW and (b) the loss in fuel cost per hour if the load is equally shared by both units. .