Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL INTEGRATED CIRCUITS				      	     Time : 3 hours

Subject Code:	IT212								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Binary equivalent for the decimal number (25.5)10  is   ____________.

2.	The Excess-3 code of 1001 __________.

3.	What are universal gates?

4.	The AND operation can also be produced with __________.

a.	two NAND gates    			b.	three NAND gates

c.	one NOR gate   				d.	three NOR gates

5.	How many flip flops are required to design a Mod-10 counter?

6.	Draw D flip flop and its truth table.

7.	What is fan in and fan out?

8.	What are the types of TTL logic?

9.	In PAL, only __________ gates are programmable.

10.	Distinguish between EPROM and EEPROM.

PART – B  (5 x 3 = 15 MARKS)

11.  Simplify the following Boolean expression: AB’C’+A’B’C’+A’BC’+A’B’C.

12.	Design a half subtractor circuit.

13.	Distinguish between latch and flip flop.

14.	State the salient features of ECL family.

15.	Compare PROM, PAL and PLA.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Convert the following

i.     (53.625)10 into equivalent binary number.

ii.	(110010.1110)2  into equivalent decimal number.

iii.	(268)10 into equivalent octal number.

iv.   (615)8 into equivalent hexa decimal number	.							  	  (12)

b. 	Realize the following Boolean function using only NAND gates.

Y=AB+A’B’C’.           													    	    (3)

(OR)

17.	Reduce the following function using K-map technique.

F(W, X, Y, Z) = ∑(0,2,3,6,7,8,10,11,12,15).

18.	Design and draw the logic diagram of binary to gray code converter with the help of its truth table and K- map simplification.

(OR)

19.	a. 	Explain in detail about  magnitude comparator with a neat circuit diagram and truth table. 																				    (8) b. 	Implement the following Boolean function with 8:1 multiplexer.

F(A, B, C, D) = ∑m(0, 2, 6, 10, 11, 12, 13) + d(3, 8, 14)  							    (7)

[P.T.O]

20.	a. 	Draw and explain the logic diagrams of JK and T flip flops. Also derive its characteristics

equations.																	   (8)     	b. 	Design a MOD- 3 counter using JK flip flops.									   (7)

(OR)

21.	Describe the types of shift registers and its operations with necessary diagrams.

22.	Write short notes on

a. 	RTL

b. 	DTL

(OR)

23. 	a. 	Draw and explain the basic CMOS Inverter circuit.								  (10)

b. 	Compare the totem-pole and open-collector outputs.								    (5)

24.	Explain the following:

a. 	PAL

b.	PLA

(OR)

25.	Draw and explain the block diagram of Random access Memory with necessary examples.