Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	ELECTRONIC DEVICES AND BASIC CIRCUITS	      	     Time : 3 hours

Subject Code:	MT224								 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	Define signal.

2.  	What is the function of an amplifier?

3.  	Define CMRR.

4.  	The open loop gain of an ideal OP-AMP is __________.

5.  	Mention any two applications of diodes.

6.  	What is the function of diode?

7.  	Name the two types of FET.

8. 	Mention any two applications MOSFET.

9.  	What are the different configurations of BJT?

10. 	What is a bipolar transistor?

PART – B  (5 x 3 = 15 MARKS)

11.  Define Voltage Gain of a linear amplifier.

12.  Define Voltage follower.

13.  Distinguish between zener breakdown and avalanche breakdown.

14.  Write a short note on CMOS inverter.

15.  Draw the circuit symbols of  NPN and PNP transistor.

PART – C  (5 x 15 = 75 MARKS)

16.  	Explain the four  amplifier models in detail.

(OR)

17.  	Describe the Digital Logic Inverter with necessary circuits in detail.

18.	a. 	Explain how an Op-Amp can be used as an integrator.							    (8)

b. 	Explain how an Op-Amp can be used as a differential amplifier.					    (7)

(OR)

19.  	In a non-inverting amplifier circuit using Op-Amp, given R1= 5 kΩ, Rf  = 20 kΩ and Vi = 1V.  A load resistor of 5 kΩ is connected at the output.  Draw the circuit diagram. Calculate (i) V0  ii) A CL   iii)the load current IL iv) the output current I0 indicating proper direction of current flow.

20.	a. 	Explain the V-I characteristics of a pn junction diode								    (6)

b. 	Explain the application of a diode as a i)Rectifier		ii)OR gate  iii)AND gate.		    (9)

(OR)

21.	a. 	Explain the operation of special types of diodes.									    (8)

b. 	Explain the operation of Zener diode as a voltage regulator.						    (7)

22.  	Explain the operation of MOSFET as a   common source amplifier and a switch.

[P.T.O]

(OR)

23.   Draw the structure of depletion MOSFET and explain in detail.

24.	a. 	Explain the operation of BJT as a switch.										    (7)

b. 	Explain the operation of a single stage CE amplifier with a neat diagram.			    (8)

(OR)

25.  What is the need for biasing in BJT amplifier circuits?  Describe the fixed bias and voltage  	divider methods of biasing with circuit diagrams.