Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FUNDAMENTALS OF ELECTRONIC COMMUNICATION													      	     								     Time : 3 hours

Subject Code:	MT233								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Signal to Noise Ratio.

2.	What is Q factor?

3.	Define oscillator.

4.	What is Intermediate frequency?

5.	What is the bandwidth of Amplitude Modulated wave?

6.	Define modulation index of a frequency modulated wave.

7.	Mention any one angle modulator circuit.

8.	What are the indirect methods of achieving Frequency Modulation?

9.	What is a bit?

10.	Mention any two block codes.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Equivalent Noise temperature of the amplifier.

12.	Define image signal.

13.	Draw the frequency spectrum of Amplitude Modulated wave.

14.	Define Pulse Amplitude Modulation.

15.	Define forward error correction.

PART – C  (5 x 15 = 75 MARKS)

16.	a.  	Calculate the thermal noise power available from any resistor at room temperature (290K) 	for  a bandwidth of 1 MHz.  Calculate the corresponding noise voltage, given that R =50	 	ohms.																		    (3)

b.  	Two resistors of 20 and 50 kohms are at room temperature (290K).  For a bandwidth of 100kHz, calculate the thermal noise voltage generated by (a) each resistor, (b) the two resistors in series, and (c) the two resistors in parallel.							    	    (7)

c.	A mixer stage has a noise figure of 20dB and this is preceded by an amplifier that has a noise figure of 9 dB and an available power gain of 15 dB.  Calculate the overall noise figure referred to the input.													    (5)

(OR)

17.	a.	Explain the operation of tuned RF amplifier.   									    (8)

b.	Explain the operation of broadband video amplifiers.								    (7)

18.	a.	Explain the working of a crystal oscillator with a neat diagram.					    (8)

b.	Explain frequency synthesizer with a neat block diagram.							    (7)

(OR)

19.	a.	Explain the operation of a superheterodyne receiver with a neat block diagram.		  (10)

b.	Explain adjacent channel selectivity.											    (5)

[P.T.O.]

20.	Define amplitude modulation. Derive the equation of Amplitude Modulated wave. Draw the frequency spectrum of AM wave.

(OR)

21.	Explain SSB generation using

a.	Filter Method																    (5)

b.	Phase – shift Method and 														    (5)

c.	The third Method. 															    (5)

22.	a.	Define PAM. Explain how PAM is generated.  Draw the spectrum of natural PAM.	    (8)

b.	Explain Time Division Multiplexing.											    (7)

(OR)

23.	Explain the following with a neat diagram.

a.	Foster Seely discriminator.													    (8)

b.	Ratio detector.																    (7)

24.	Explain probability of bit error in baseband transmission.

(OR)

25.	Explain block codes.