Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS    	     Time : 3 hours

Subject Code:	MT240								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the function of SID and SOD pins in 8085?

2.	List some of the logical instructions in 8085.

3.	What is the function of MN/MX pin?

4.	What is the use of Instruction Queue in 8086 microprocessor?

5.	What is the purpose of the 8255 PPI?

6.	What is an USART?

7.	What is interfacing?

8.	List the major components of the keyboard/Display interface.

9.	What is Microcontroller?

10.	What is the function of DPTR register?

PART – B  (5 x 3 = 15 MARKS)

11. 	Define (a) Instruction Cycle (b) Machine Cycle (c) T-state.

12. 	How is the physical address calculated in 8086 microprocessor? Give an example.

13. 	What are the functional types used in control words of 8251A?

14.	List the major components of 8251A programmable communication interface.

15. 	What is the advantage of microcontroller over microprocessor?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain various addressing modes of 8085 microprocessor with example.

(OR)

17. 	Write an ALP to perform logical operations using 8085 microprocessor.

18.	Explain the internal hardware architecture of 8086 microprocessor with neat diagram.

(OR)

19.	Explain Data transfer, arithmetic and branch instructions of 8086 microprocessor with suitable example.

20.	Explain the process of the Direct Memory Access (DMA) and the functions of various elements of the 8237.

(OR)

21.	With neat block diagram explain the 8251 and its operating modes.

22.	With neat diagram explain the interfacing of a Microprocessor to Keyboard.

(OR)

23. 	With neat diagram explain the interfacing Analog to Digital Data Converters with microprocessor.

24.	Explain the architecture of 8051 with its diagram.

(OR)

25.	With example explain the arithmetic and logic instruction of 8051 microcontroller.