Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MASS TRANSFER OPERATIONS					      	     Time : 3 hours

Subject Code:	09BT206/12BT203/BT236						       Maximum Marks: 100

#### Answer ALL questions

(Allowed the mass transfer data book and extra graph sheets)

PART – A  (10 x 1 = 10 MARKS)

1. 	Give the driving force in Mass transfer operations.

2. 	Write the Stefan’s equation.

3.  	Give your suggestion how to separate the same boiling point components.

4.  	Write the equations for internal reflux ratio and external reflux ratio.

5. 	Hyper sorption refers to __________ process.

6. 	Draw a  batch process absorption block diagram..

7. 	Define different types of  packing methods..

8.  	Give the different adsorbents.

9. 	What is meant by soultropic system?

10. 	Define Extraction and leaching.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between the diffusion and eddy diffusion.

12. 	Define: Murphee plate efficiency, Point efficiency and Overall efficiency.

13. 	What are the  merit and demerit on packing methods in absorption column?

14. 	Differentiate between Physical and chemical adsorption.

15.  What are leaching and extraction?

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain an expression for mass flux under steady state diffusion of A through a stagnant

gas  Film B.

(OR)

17. 	Methane diffuses at a steady state through a tube containing Helium at point 1, the pressure is 15.55 KPa. At point 2, which is 30 mm apart, the partial pressure of Methane is10 kPa? The total pressure is 101.32 KPa and the temperature is 293K. At this pressure and temperature the diffusivity is 6.75x10-5 m2/s. calculate

a.	The flux of methane at steady state for equimolar counter diffusion.

b.	The partial pressure at 2 cm from point 1.

18. Explain how the number of stages are determined using Mc Cabe Thiele method for a 	specified reflux ration with neat sketch.

(OR)

19. 	Derive Rayleigh’s equations and flash distillation with suitable sketch.

20.  Explain the design of absorption column with neat sketch.

(OR)

[P.T.O]

21. 	Ammonia – air mixture containing 2 % Ammonia at 20 °C and 1 atmosphere is to be scrubbed with water in a sieve plate column. The water  rate is 32.6 kg / square meter  sec. 98 % Ammonia  is to be recovered .Calculate the number of ideal stages assuming  1.5  times that of min flow rate, isothermal operation. (Constant temperature)

| PA mmHg | 12 | 18.1 | 31.7 | 50 | 69.6 | 10 |
| --- | --- | --- | --- | --- | --- | --- |
| CA gm of NH3/100gm of Water | 2 | 3 | 5 | 7.5 | 10 | 20 |

22. 	Derive and explain the single stage adsorption process with suitable sketch.

(OR)

23. 	Explain the principles and equilibria of ion exchange operation.

24. 	Explain with neat sketches, different equipments used for liquid – liquid extraction.

(OR)

25. 	Write short notes on

a.	Heap leaching 	b.	Solution Mining

c.	Shanks system 	d.	Equilibrium curves in leaching