Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MECHANICS OF FLUIDS							      	     Time : 3 hours

Subject Code:	09CE202 / 12CE205 / CE277		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Compare dynamic viscosity and kinematic viscosity.

2.	Calculate the specific gravity of one litre of crude oil weighing 9.6 N.

3.	What is metacentre?

4.	Distinguish between laminar and turbulent flow.

5.	Define “velocity potential function”.

6.	What is hydraulic gradient line?

7.	Mention where Cipolletti weir is used in practice.

8.	Highlight the assumptions made in the derivation of Bernoulli’s equation.

9.	Define the term ‘dimensional analysis’.

10.	State Buckingham’s π Theorem.

PART – B  (5 x 3 = 15 MARKS)

11.	The velocity distribution for flow over a flat plate is given by u=32y−y32, where u is the point velocity in m/sec at a distance ‘y’ metre above the plate. Determine the shear stress at

y = 9 cm. Assume dynamic viscosity as 8 poise.

12.	What do you mean by equipotential line and a line of constant stream function?

13.	Derive the Chezy’s formula for loss of head due to friction in pipes.

14.	A jet of water, issuing from a sharp-edged vertical orifice under a constant head of 10.0cm, at a certain point, has the horizontal and vertical co-ordinates measured from the vena-contracta as 20.0cm and 10.5cm respectively. Find the value of Cv. Also find the value of Cc if Cd = 0.60.

15.	Explain the terms: distorted models and undistorted models.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	An oil film of thickness 1.5mm is used for lubrication between a square plate of size 0.9m 	x 0.9m and an inclined plane having an angle of inclination 20˚. The weight of the 	square plate is 392.4N and it slides down the plane with a uniform velocity of 0.2m/sec. 	Find the dynamic viscosity of the oil. 											    (9)

b.	Derive the relation between surface tension and internal pressure of a droplet of water and 	a soap bubble.                 														    (6)

(OR)

17.	A cylindrical gate of 4m diameter 2m long has water on its both sides as shown in figure. Determine the magnitude, location and direction of the resultant force exerted by the water on the gate. Find also the least weight of the cylinder so that it may not be lifted away from the floor.

[P.T.O]

A

Water Surface

Fy1

Fy2

D

Water Surface

Water surface

4m

B

Fx1

Fx2

D

18.	State Bernoulli’s equation for the flow of an incompressible fluid. Derive an expression for Bernoulli’s theorem from first principle and state the assumptions made for such a derivation.

(OR)

19.	In a two dimensional incompressible flow, the fluid velocity components are given by u = x – 	4y and v = - y - 4x. Show that the velocity potential exists and determine its form as well as 	stream function.

20.	Derive an expression for Hagen Poiseuille’s formula.

(OR)

21.	Three pipes of 400mm, 200mm and 300mm diameters have lengths of 400m, 200m and 300m respectively. They are connected in series to make a compound pipe. The ends of this compound pipe are connected with two tanks whose difference of water levels is 16m. If co-efficient of friction for these pipes is same and equal to 0.005, determine the discharge through the compound pipe neglecting first the minor losses and then including them.

22.	a.	Derive an expression for the discharge through a Venturimeter.					    (5)

b.	A 20cm x 10cm Venturimeter is provided in a vertical pipe line carrying an oil of specific gravity 0.8, the flow being upwards. The difference in elevation of the throat section and entrance section of the Venturimeter is 50 cm. The differential U tube mercury manometer shows a gauge deflection of 40cm. Calculate: i) the discharge of oil and ii) the pressure difference between throat section and entrance section. Take Cd = 0.98 and specific gravity of mercury as 13.6.															  (10)

(OR)

23.	a.	Derive an expression to determine the discharge through a rectangular notch. 		  (10)

b.	Find the discharge over a rectangular weir of length 80m. The head of water over the weir is 1.2m. The velocity of approach is given as 1.5m/sec. Take Cd = 0.6.				    (5)

24.	Assuming that rate of discharge Q of a centrifugal pump is dependent upon the mas density ρ of fluid, pump speed N (rpm), the diameter of the impeller D, the pressure p and the viscosity of the fluid µ. Show using Buckingham’s π theorem, that it can be represented by

Q=ND3 φgHN2D2 , νND2

Where  H = head and υ = kinematic viscosity of the fluid.

(OR)

25.	Define and explain Reynolds’s number and Froude’s number and derive expressions for the two numbers.