Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MECHANICS OF DEFORMABLE BODIES II/ ADVANCED STRENGTH OF MATERIALS										    Time : 3 hours

Subject Code:	09CE206/12CE210/CE241						      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define slope at a point in a deflected beam.

2. 	What are conjugate beams?

3. 	Differentiate between strut and column.

4. 	Define bulking load.

5. 	Maximum Principal stress theory is also known as __________.

6.  	Differentiate between thick cylinder and thin cylinder.

7. 	What is the other name of shear centre?

8. 	The analysis of curved beam is done with __________ theory.

9. 	Define unsymmetrical bending.

10. 	Give the reasons for unsymmetrical bending.

PART – B  (5 x 3 = 15 MARKS)

11. A cantilever of length 3 m is carrying a udl of 5 kN/m throughout its length. If the moment of inertia of the beam is 1x108 mm4 and the value of E = 2.1x105N/mm2. Find the deflection at the free end.

12.  State any three assumptions made in the Euler’s theory.

13.  Name any three important theories of failure.

14. 	Define shear centre.

15. Explain the expression used for finding out the resultant bending stress at a point due to unsymmetrical bending.

PART – C  (5 x 15 = 75 MARKS)

16. 	A beam AB of span 4m span is simply supported at the ends and is loaded as shown in diagram. Determine  a) Deflection at C   b) Maximum deflection and  c) Slope at the end A

Take E= 200GPa and I= 20x10-6 m4.

(OR)

[P.T.O]

17. 	For the beam shown in the figure, determine the following:

a. 	Slope at A 		b.	Maximum deflection.  Take  I = 8 x 10-5 m4,   E = 200 x 106 kN/m2.

18.  Derive the expression for Euler`s Crippling load when both ends of the column are hinged.

(OR)

19.  	A hollow cast iron column 200mm outside diameter and 150mm inside diameter, 8m long has both ends fixed. It is subjected to an axial compressive load. Taking a factor of safety as 6 and α = 1/1600, determine the safe Rankine’s load.  c=560MPa.

20.  	A compound cylinder, formed by shrinking one tube to another is subjected to an internal pressure of 90 MN/m2. Before the fluid is admitted, the internal and external diameters of the compound cylinder are 180mm and 300mm respectively and the diameter at the junction is 240mm. If after shrinking on, the radial pressure at the common surface is 12 MN/m2, determine the final stresses developed in the compound cylinder.

(OR)

21.  	Give detailed note on any five theories of failure.

22. 	A channel section has flanges 120 X 20 mm and web 160 X 10 mm. Determine the shear centre of the channel.

(OR)

23. 	A crane hook carries a load of 7.5kN and the load line is at a distance of 20mm from the inner edge of the section which is trapezoidal. The load line is also passes through the centre of curvature of the hook. The dimensions of the central horizontal trapezoidal section are: inner width=30 mm, outer width= 15mm. Calculate the maximum and minimum stresses.

24.  	A beam of ‘T’ section (flange 100x20mm, web 150x10mm) is 2.5m long and is simply supported at the ends. It carries a load of 3.2kN inclined at 20o to the vertical and passing through the centroid of the section. If E = 200GPa, calculate the maximum tensile and maximum compressive stress.

(OR)

25. 	A cantilever of I section is 2.4m long. It is subjected to a load of 200N at the free end. The load is inclined at an angle of 20 ͦ with the vertical plane which passes through the centre of gravity. Find out the resultant stresses at the corner points of top and bottom flanges.