Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	STRUCTURAL ANALYSIS - I						      	     Time : 3 hours

Subject Code:	09CE208/12CE214/CE254		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.    Write the conditions for static indeterminacy.

2.	What is the degree of freedom of a cantilever beam?

3.	Write any one use of influence line diagram.

4.	State Eddy’s theorem.

5.	What do you mean by rise in an arch?

6.	Write any 2 differences between 2 hinged and 3 hinged arches.

7.	Define Tension Coefficient of a truss member.

8.	What is the function of suspenders in a suspension bridge?

9.	What is the other name of force method?

10.	Write the equation of theorem of three moments.

PART – B  (5 x 3 = 15 MARKS)

11.	State Castigliano’s First theorem.

12.	State Muller-Breslau principle.

13.	What is a linear arch?

14.	What is suspension bridge? What are elements of the same?

15.  List the process of analysis of structures by consistent deformation method.

PART – C  (5 x 15 = 75 MARKS)

16.	Using Castigliano’s method, obtain the deflection under the point load of 60 kN applied at 3m. from the left support of a simply supported beam of span 4m. EI = 2.2 MNm2.

(OR)

17.	Using the principle of virtual work, obtain the deflection under the point load of 60 kN applied at 3m from the left support of a simply supported beam of span 4m. EI = 2.2 MNm2.

18.	A uniformly distributed load of 1 kN per meter run, 6m long crosses a girder of 16m span. Construct the maximum SF and BM diagrams and calculate the values at section 3m, 5m and 8m from the left hand support.

(OR)

19.	A set of four concentrated loads 100kN, 80kN, 60kN and 80kN at a fixed intervals of 3m, 2m and 2m, move from left to right over a simply supported beam of 18 m. span. 100 kN is the leading load. Using ILD determine   (a)Absolute max. shear forces. (b) Absolute max. Bending moment in the beam.

20.	A symmetrical three hinged parabolic arch of span 40 m and rise 8 m carries a UDL of 30kN/m over the left half of the span. Calculate the reactions at the supports. Also calculate the bending moment, radial shear and normal thrust at a distance of 10 m from the left support.

(OR)

[P.T.O]

21.	A three hinged circular arch of span 16 m and rise 4 m is subjected to two point loads of 100kN and 80kN at the left and right quarter span points respectively. Find the reactions at the supports. Find also the bending moment, radial shear and normal thrust at 6 m from left support.

22.	Plan of a tripod is shown in Figure. The feet A, B and C of the tripod are in the same horizontal plane and the apex D is 3.75 m. above the plane. Horizontal loads of 100kN and 150kN are applied at D as shown. Using method of tension coefficients, find the forces in all the members assuming that all joints are pin joints.

(OR)

23.	A cable of span 100 m has its ends at heights 8 m and 15 m above the lowest point of the cable.  It carries a uniformly distributed load of 10 kN/m per unit horizontal run of the span.  Determine the horizontal and vertical reactions at the support.  What is the length of the cable?

24.	Analyze the continuous beam ABC shown in the figure below using three-moment equation and draw the bending moment diagram by Force method.  EI is constant throughout.

C

6 m

8 m

40 kN/m

B

A

(OR)

25.	A two span beam ABC, fixed at the ends A and C; AB = 8 m, BC = 5 m. The entire span AB carries a UDL of 5 kN/m and the span BC carries a clockwise moment of 80 kNm at 2 m from C. Plot BMD and SFD. Use Clapeyron’s theorem of three moments.