Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DESIGN OF STEEL STRUCTURES  (LIMIT STATE METHOD)   / DESIGN OF STEEL STRUCTURES						     Time : 3 hours

Subject Code:	09CE219/12CE223/CE252						       Maximum Marks: 100

#### Answer ALL questions

(IS 800-2007, IS 875 and steel tables are permitted)

PART – A  (10 x 1 = 10 MARKS)

1. 	Minimum thickness of weld ___________.

2. 	Sectional classification of steel sections based on ___________.

3. 	Tension member may fail due to ___________.

4. 	Strength of compression member depends on ___________.

5. 	Beam sections to be selected based on ___________.

6. 	Built-up sections are provided to increase the ___________.

7. 	Thickness of base plate is determined based on ___________.

8. 	Stiffeners are provided in plate girders to resist the ___________.

9. 	IS Code to be used to find the wind pressure ___________.

10. 	Minimum live load to be considered for the design of truss ___________.

PART – B  (5 x 3 = 15 MARKS)

11. Write the reasons for the failure of bolted connections.

12. Define Block shear.

13. Define Lateral torsional buckling.

14. Write the components of plate girder.

15. List the different types of roof truss.

PART – C  (5 x 15 = 75 MARKS)

16.	A tie member in a truss girder in 250mmX14mm in size. It is welded to a 10mm thick gusset plate by a fillet weld. The overlap of the member is 300mm and the weld size is 6mm. Determine the design strength of the joint. Assume shop welding. What is the increase in strength of the joint, if welding is done all around?

(OR)

17. 	Calculate the strength of 20mm bolt of Grade 4.6, the main plates to be jointed are 12mm thick. (Lap joint)

[P.T.O]

18. 	Compute the tensile strength of an angle section ISA 150X115X8mm of Fe410 grade of steel connected with the gusset plate for the following cases a) Gross section yielding b) Net section rupture.

(OR)

19. 	Design the column to support the factored load of 1050kN, the column has an effective length of 7m with respect to Z axis and 5m with respect to Y axis. Use steel of grade Fe410.

20. 	Design the Laterally restrained simply supported beam for the following data Span -4m, UDL - 10kN/m (including self wt), Fe410 grade steel.

(OR)

21. 	Design the column in a frame with flexible joints of 3.2m height and subjected to the axial load of 1275kN, Moment at top 22.5kNm, Fe410 grade steel.

22. 	Explain different components of welded plate girder and its behavior with neat sketches.

(OR)

23. 	A column section ISHB 350@710.2N/m is subjected to following factored loads. Axial load 1500 kN, Assuming M20 grade concrete for the pedestal and a square base plate, design the followings.

a) Thickness of base plate 		b) Anchor bolts.

24. 	Design a purlin on a sloping roof truss with a dead load of 300N/m and wind load (normal to the roof) of 3000N/m. The purlins are 1.5m center to center and of span 6m, simply supported on a rafter at a slope of 30 degrees.

(OR)

25. 	Briefly explain the Loads and load combinations to be used for the analysis and design of roof truss.