Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	FLUID MECHANICS AND MACHINERY			      	     Time : 3 hours

Subject Code:	09CE242/12CE263/CE212						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	The weight per unit volume of a fluid is called as __________.

2.  	Define kinematic viscosity.

3.  	Fluids are incompressible. (True/False)

4.  	The viscosity of an ideal fluid is __________.

5.  	Write an application of Venturimeter.

6.  	Differentiate a notch and a weir.

7.  	The force exerted by a jet of water of diameter 75 mm on a stationary flat plate, when the jet	strikes the plate normally with velocity of 20 m/s is __________.

8.   	If the suction head is 10 m and delivery head us 20 m then the vaue of static head is __________.

9.   	Define jet ratio.

10. 	If the shaft power of a pelton wheel is 15450 kW and power at the inlet of turbine is 21818 kW	then the value of overall effeciency of the pelton wheel is __________.

PART – B  (5 x 3 = 15 MARKS)

11. Find the surface tension in a soap bubble of 40 mm diameter when the inside pressure is 2.5 N/m2 above atmospheric pressure.

12. An open circular tank of 20 cm diameter and 100 cm long contains water upto a height of 60

cm. The tank is rotated about its vertical axis at 300 r.p.m., find the depth of parabola formed

at the free surface of water.

13. Find the head lost due to friction in a pipe of diameter 300 mm and length 50 m, through  which the  water is flowing at a velocity of 3 m/s, using Darcy formula. Take kinematic visosity for water as 0.01 stoke.

14. What are the applications of air vessels?

15. Explain the working of Gear pump.

PART – C  (5 x 15 = 75 MARKS)

16. A square plate of size 1 m × 1m and weighing 350 N slides down an inclined plane with a 	uniform velocity of 1.5 m/s. The inclined plane is laid on a slope of 5 vertical to 12 horizontal 	and has an oil film of 1 mm thickness. Calculate the dynamic viscosity of oil.

(OR)

17. 	Calculate the capillary rise in a glass tube of 3 mm diameter when immersed vertically in

(a) water and (b) mercury. Take surface tensions for mercury and water as 0.0725 N/m and 	0.52 N/m respectively in contact with air. Specific gravity for mercury is given as 13.6.

18. 	Check if φ = x2 −y2 + y represents the velocity potential for a two dimensional irrotational

flow.  If it does then determine the stream function ψ.

(OR)

[P.T.O]

19. 	A 40 cm diameter pipe, conveying water, branches into two pipes of diameters 30 cm and 20cm respectively. If the average velocity in the 40 cm diameter pipe is 3 m/s. Find the discharge in this pipe. Also determine the velocity in 20 cm pipe if the average velocity in 30 cm diameter pipe is 2 m/s.

20. 	An	 orifice meter with orifice diameter 15 cm is inserted in a pipe of 30 cm diameter. The pressure gauges fitted upstream and downstream of the orifice meter give readings of 14.715 N/cm2 and 9.81 N/cm2 respectively. Find the rate of flow of water through the pipe in liters/second. Take coefficient of discharge as 0.6.

(OR)

21. 	Determine the rate of flow of water through a pipe of diameter 10 cm and length 60 cm when one end of the pipe is connected to a tank and other end of the pipe is open to the  atmosphere. The height of the water in the tank form the centre of the pipe is 5 cm. Pipe is given as horizontal and value of coefficient of friction is 0.01. Consider minor losses.

22. 	A plate is acted upon at its centre by a jet of water of diameter 20 mm with a velocity of 20m/s. The plate is hinged and is deflected through an angle of 15 o. Find the weight of the plate. If the plate is not allowed to swing, what will be the force required at the lower edge of the plate to keep the plate in vertical position.

(OR)

23. 	Find the number of pumps required to take water from a deep well under a total head of 156 m.  	Also the pumps are identical and are running at 1000 r.p.m. The specific speed of each pump is 	given as 20 while the rated capacity of each pump is 150 liters/second.

24. A Kaplan turbine runner is to be designed to develop 7357.5 kW shaft power. The net available 	head is 10 m. Assume that the speed ratio is 1.8 and flow ratio is 0.6. If the overall efficiency is 	70% and the diameter of the boss is 0.4 times the diameter of the runner, find the diameter of 	the 	runner, its speed and specific speed.

(OR)

25. 	A pelton wheel has a mean bucket speed of 35 m/s with a jet of water flowing at the rate of 	1m3 /s under a head of 270 m. The buckets deflect the jet through an angle of 170o. Calculate 	the power delivered to the runner and the hydraulic efficiency of the turbine. Assume 	coefficient of velocity as 0.98.