Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MECHANICS OF SOLIDS							      	     Time : 3 hours

Subject Code:	09CE245 / 12CE202 / 12CE264 / CE203			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Stress.

2.	Define Poisson’s ratio.

3.	What do you mean by thin cylinders?

4.	Define point of contraflexure.

5.	Write down the simple bending equation.

6.	How springs are classified?

7.	Write the types of beams.

8.	Define torsion

9.	What is the advantage of Macaulay’s method for finding beam deflections?

10.	What do you mean by Effective Length?

PART – B  (5 x 3 = 15 MARKS)

11.	State Hooks law.

12.	Explain shear force and bending moment diagrams.

13.	Write any four assumptions in the theory of simple bending.

14.	Explain maximum shear  stress theory.

15.	State any three assumptions made in the Euler’s long column theory.

PART – C  (5 x 15 = 75 MARKS)

16.	A circular bar ABCD is rigidly fixed at A and D and is subjected to axial forces as shown in the figure below. Determine the reactions and forces in each portion of the bar and the displacement of point B and C. Take E = 200 kN/mm2.

(OR)

17.	 In a tensile test on a hollow tube of external diameter 20mm and internal diameter 16mm, an axial load of 1500N produced an elongation of   0.0038mm in a length of 75mm while diameter is contracted by 0.0029mm. Calculate the Poisson’s ratio, Young’s modulus and bulk modulus.

18.	A cantilever beam of 2 m long carries a uniformly distributed load of 1.5 kN/m over a length of 1.6 m from the free end. Draw the shear force and bending moment diagrams for the beam.

(OR)

[P.T.O]

19.	Draw the shear force and bending moment diagrams for the overhanging beam carrying uniformly distributed load of 2 kN/m over the entire length and a point load of 2 kN as shown in the figure below. Also locate the point of contraflexure.

20.	The principal stresses at a point across two perpendicular planes are 75MN/m2, (tensile) and 35MN/m2 (tensile). Find the normal, tangential and the resultant stresses and its obliquity on a plane at 200 with the major principal plane.

(OR)

21.	Using Mohr s circle method, determine the normal and tangential stresses across the oblique plane. Take major and minor principal stresses as 65 N/mm2  and 35 N/mm2,shear stress 25 N/mm2, oblique angle (θ=450).

22.	A close coiled helical spring has a stiffness of 5N/mm. Its length when fully compressed with adjacent coils touching each other is 40 cm. The modulus of rigidity of the material of the spring is 8x104 N/mm2. Determine the wire diameter and mean coil diameter if their ratio is 1/10. What is the corresponding maximum shear stress in the spring?

(OR)

23.	A hollow shaft of diameter ratio 3/8 is required to transmit 600kW at 110 rpm, the maximum torque being 20% greater than the mean. The shear stress is not to exceed 63 MN/m2 and the twist in a length of 3m not to exceed 1.4 degrees. Calculate the maximum external diameter satisfying these conditions.

24.	Derive the Euler’s formula when both ends of the column are hinged.

(OR)

25.	A timber joist of 4m span has to carry a total load of 80 KN uniformly distributed over the length. Calculate the dimensions of the joist section if the maximum permissible stress is limited to 8 N/mm2. The ratio of depth to width is to be 1.50.