Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	ENGINEERING MECHANICS						      	     Time : 3 hours

Subject Code:	09CE247 / 12CE203 / CE204 / CE242				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give the reaction for a roller support and a fixed support.

2. 	What is a plane force?

3. 	What are the equilibrium conditions to be satisfied?

4. 	What are the different types of loads acting on a beam?

5. 	What do you mean by a cable?

6. 	What is the nature of forces in a truss?

7. 	What do you mean by rectilinear motion?

8. 	Define kinetic energy.

9. 	What do you mean by impact?

10. 	Define the principle of conservation of energy.

PART – B  (5 x 3 = 15 MARKS)

11. Find the resultant of the two concurrent forces acting.

450

300

x

y

60 N

100 N

12. 	Brief Lami’s theorem with a neat sketch.

13. 	Find the centroid of trapezium shown in figure.

30cm

10cm

20cm

14. 	A particle dropped from the top of a tower strikes the ground after 4 seconds.  Find the velocity of the stone as t hits the ground and the height of the tower.

15. 	Explain the law of conservation of momentum.

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16. 	Determine the support reactions for the simply supported beam shown.

80N

1m

100Nm

60N/m

6m

3m

A

B

(OR)

17. 	An electric lamp weighing 20 N hangs from a point C by two strings AC and BC as shown. Find the forces in the strings using Lami’s theorem and by equation of statics.

300

600

B

C

A

18. 	A ladder 4 m long leans against a smooth vertical wall at an angle 60° with the horizontal as shown in figure. The weight of the ladder is 700 N. When a person weighing 600N stands on the ladder 1.2 m from the bottom, the ladder is just about to slide. Calculate the coefficient of friction between the ladder and the floor.

(OR)

300

W2

W1

P

cable

19.  	A block of weight W1 = 500N rests on a horizontal surface and supports another block of weight W2 = 300N on top of it as shown in Figure.  Find the Force ‘P’ applied to the lower block and the tension in the cable. Coefficient of friction for all contact surfaces to be 0.3.

20. 	Determine the moment of inertia about the centroidal axes of the I-section shown in Figure.

(OR)

21. Determine the forces in the various members of the pin jointed truss using method of joints.

A

1.0 m

3 kN

3kN

B

E

D

C

1.5m



1.0m



22. 	A car starts with a velocity of 3m/s and moves in a straight line with a constant acceleration.  If the velocity at the end of the 5 seconds is 5.5m/s find a) the constant acceleration b) distance travelled in 10 seconds c) velocity when it has travelled a distance of 72m.

(OR)

23. 	A boat of weight 5000N is at rest in a river.  A man weighing 800 N jumps into the boat with a velocity of 1.5m/s.  With what velocity the boat will move immediately after the man has jumped. Use law of conservation.

24. 	A sphere of weight 10 N moving horizontally at 3 m/s collides with another sphere of weight 50 N moving in the same direction at 0.6 m/s.  If after impact, the 50N continuous to move in the same direction with a velocity of 1.3m/s, what is the velocity of the first sphere after impact? Also, determine the coefficient of restitution.

3 m/s

0.6 m/s

10 N

50 N

Line of impact

(OR)

25. 	A projectile is fired with an initial velocity of 60m/s inside a tunnel which is 5m high.  Find the angle of projection and maximum possible horizontal range and the time of flight.