Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DIGITAL PRINCIPLES AND SYSTEM DESIGN		      	     Time : 3 hours

Subject Code:	09IT204/12IT206								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	x+x'=___________.

2.	What is a K-map?

3.	Represent 1234 in Excess-3-code.

4.	What is HDL?

5.	How many selection lines and input output data are needed for the 4 X 1 Multiplexer?

6.	A decoder with an enable input can function as a ___________.

7.	The 4-bit shift register is initialized to 1011. What are the contents of the register   after it is shifted six times to the right, with the serial input being 101111?

8.	___________ is a one bit storage element

9.	What is a primitive flow table?

10.	How to avoid critical races?

PART – B  (5 x 3 = 15 MARKS)

11.	P.T AB'+AB=A .

12.	What is the output of the following circuit?

13.	What are the different types of ROM?

14.	Explain the operation of D latch.

15.	What is a flow table? Give example.

PART – C  (5 x 15 = 75 MARKS)

16.	Simplify the following Boolean functions using K map

a.	F=∑(0, 4, 6, 8, 10, 15).			                                               					    (8)

b.	F=∑ (0, 1, 3, 5, 6, 9, 11, 12, 13, 15)                                                         				    (7)

(OR)

17.	a.	Express the following function using sum of minterms and product of maxterms:

F(A,B,C,D ) = B’D+A’D+BD   												  (10)

b.	Find the complement of  xy'+x'y.												    (5)

18.	Explain the Half adder and full adder.

(OR)

[P.T.O]

19.	a.	Design and explain a Decimal adder circuit.										(10)

b.	Write an HDL for Half Adder circuit.                   									(5)

20.	Design a 3X8 decoder and mention its applications.

(OR)

21.	Implement the following function using PLA

a.	F1=AB’+AC+A’BC’															(8)

b.	F2=(AC+BC)’																(7)

22.	What is a shift Register? What are the different types of shift registers? Explain the Serial In Serial Out and Serial In Parallel Out shift register.

(OR)

23.	a.	Explain the operation of master slave D-flip flop.

b.	Construct a JK flip flop using the following components: D flip flop, 2-to-1 multiplexes and an inverter.

24.	Explain the analysis and design procedure of asynchronous sequential circuit.

(OR)

25.	Explain the race free state assignments.