Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	ENGINEERING MECHANICS						      	     Time : 3 hours

Subject Code:	09ME201/12ME230				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Newton’s first law of motion.

2. 	The lines of action of all forces meeting at a common point and lying in the same plane is called as ___________.

3. 	Sketch a cantilever beam with uniformly distributed load.

4. 	What are the different types of equilibrium?

5. 	Define centroid.

6. 	Moment of inertia of a circular section about its diameter (d) is _____________.

7. 	Write the work energy equation.

8. 	What is angle of repose?

9. 	Define coefficient of friction.

10. 	The diameter of wheel of a vehicle is 600 mm. The weight is 1000 kN. The coefficient of rolling resistance between the wheel and the ground is 0.35 mm. The force required to roll the wheel without slipping is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.   State Varignon’s theorem.

12.   Draw the free body diagram for the two spheres resting on inclined walls.

13.   Determine the centroid of a rectangular section by integration.

14.	 A particle moves along a straight line with variable acceleration. The displacement S measured in m is given in terms of time t as S = 4t3 + 3t2 + 6t + 2. Find the velocity of the particle after 3 seconds.

15.	Write short notes on Columb’s law of friction.

PART – C  (5 x 15 = 75 MARKS)

16.	Two spheres each of weight 500 N and of radius 100 mm rest in a horizontal channel of width of 360 mm as shown in the figure below. Find the reactions on the points of contact A, B and C. Assume all the surfaces of contact are smooth.

(OR)

[P.T.O]

17.	The four coplanar forces are acting at the point O, as shown in figure determine the resultant in magnitude and direction.

18.	Determine the resultant of the force systems shown in the figure.

(OR)

19.	The tension in cables AB and AC are 100N and 120N respectively as shown in figure below. Determine the magnitude of the resultant force acting at A.

20.	Locate the centroid of the sectioned area shown in the figure.

(OR)

21.	Find the moment of inertia of the section shown in the figure

22.	Two weights 80 N and 20 N are connected by a thread and move along a rough horizontal  plane under the action of force 40 N, applied to the first weight of 80 N. The coefficient of friction between the sliding surfaces of the weights and the plane is 0.3. Determine the acceleration of the weights and the tension in the thread using work energy equation.

(OR)

23.	Two blocks of weight 150 N and 50 N are connected by a string and passing through over a frictionless pulley, as shown in fig. Determine the acceleration of blocks A and B and the tension in the string.

24.	A block overlying a 10o wedge on a horizontal floor and learning against a vertical wall and weighing 1500 N is to be raised by applying a horizontal force to the wedge. Assuming co-efficient of friction between all the surfaces in contact to be 0.3, determine the minimum horizontal force to be applied to raise the block.

(OR)

25.  In a screw jack, the pitch of the square threaded screw is 5.5 mm and the mean diameter is       	70 mm. The force exerted in turning the screw is applied at the end of a lever 210 mm long 	measured from the axis of the screw. If the co-efficient of friction of the screw jack is 0.07, 	calculate the force required at the end of the lever to (a) raise a weight of 30 kN (b) lower the 	same weight.