Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	ENGINEERING THERMODYNAMICS				      	     Time : 3 hours

Subject Code:	09ME202/ME228								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	 State the Zeroth law of Thermodynamics.

2.     What are the necessary conditions for a system to be in thermodynamic equilibrium?

3. 	 What is a thermal energy reservoir?

4.     What are the causes of Irreversibility?

5.     Define dryness fraction or Quality of steam?

6.     Define compressed liquid.

7.     Explain the Daltons law of partial pressure.

8.     List the advantage of Vanderwaals equation of state.

9.     List the assumption made in the analysis of air standard cycles.

10.   What is meant by stoichiometric mixture?

PART – B  (5 x 3 = 15 MARKS)

11. 	5kg of air at 40°C is heated in a reversible non flow constant pressure process until the volume is doubled. Determine the work done.

12. 	State the Kelvin Planck and Clausius statements of second law of thermodynamics.

13. 	Determine the condition of steam at a temperature of 220°C and enthalpy of 2750kJ/kg.

14. 	Sketch the generalized compressibility chart.

15. 	Draw the P-V and T-S diagrams of Brayton cycle.

PART – C  (5 x 15 = 75 MARKS)

16.  	A piston and cylinder machine contains a fluid system which passes through a complete cycle of four processes. During a cycle, the sum of all heat transfers is -170kJ.The system completes 100cycles per minute. Complete the following table showing the method for each item, and compute the net rate of work output in kW.

| Process | Q(kJ/min) | W(kJ/min) | ΔE(kJ/min) |
| --- | --- | --- | --- |
| a-b | 0 | 2,170 | ---------- |
| b-c | 21,000 | 0 | ---------- |
| c-d | -2,100 | ---------- | -36,600 |
| d-a | ---------- | ---------- | ---------- |

(OR)

17. 	A nozzle operates under steady flow condition. At the inlet of the nozzle, the enthalpy of the fluid passing is 3000 kJ/kg and the velocity is 60m/s.At the discharge end the enthalpy is 2762 kJ/kg.The nozzle is horizontal and there is negligible heat loss from it.

i.	Find the velocity at exit from the nozzle.

ii.	If the inlet area is 0.1m2 and the specific volume at inlet is 0.187m3/kg.Find the mass flow rate.

iii.	If the specific volume at the nozzle exit is 0.498m3/kg.Find the exit area of the nozzle.

[P.T.O]

18. 	A reversible heat engine operates between two reservoirs at temperatures of 600°C and 40°C. The engine drives a reversible refrigerator which operates between reservoirs at temperatures of 40°C and –20°C. The heat transfer to the heat engine is 2000 kJ and the net work output of the combined engine refrigerator plant is 360 kJ.

a.	Evaluate the heat transfer to the refrigerant and the net heat transfer to the reservoir at 	40°C.

b.	Also find the heat transfer to the refrigerant and the net heat transfer to the reservoir at 40oC if the efficiency of the heat engine and COP of the refrigerator are each 40% of their maximum possible values.

(OR)

19. 	a.	Establish the inequality of Clausius.                                                    	  			    (8)

b.  	In a certain heat exchanger,45kg/min of water is to be heated from 60°C to 115°C by hot 	gases, which enter the heat exchanger at 225°C and flow at the rate of 90kg/min. Compute 	the net change of entropy. Assume the specific heat for water and gases as 4.18 and 1.045 	kJ/kgK.                                                                      								    (7)

20. 	Steam initially at a pressure of 15bar and 0.95 dry expands isentropically to 7.5 bar and is then throttled until it is dry. Determine per kg of steam.

a.	Change in entropy.

b.	Change in enthalpy.

c.	Change in internal energy.

(OR)

21.  	Draw and explain the phase equilibrium diagram for a pure substance on T-S Plot with      relevant constant property lines.

22. 	Find the pressure of nitrogen gas at T=175K and v=0.00375m3/kg. Using

a.	Ideal gas equation.

b.	Vander Waals equation of state.

Take the molecular weight of Nitrogen as 28.013 and Tcritical=126.2K and Pcritical= 3.39Mpa.

(OR)

23.	With a neat sketch explain the working of Bomb calorimeter.

24. 	Air is supplied at a pressure of 0.16MPa and a temperature of 40°C to an engine working on the Otto cycle. The engine compression ratio is 8.If the heat supplied to the system is 2400 kJ/kg.Calculate the maximum pressure and temperature of the cycle, the cycle efficiency, and the mean effective pressure. (For air Cv=0.718, Cp=1.005 and R=0.287 kJ/kgK).

(OR)

25. 	Draw the T-s and h-s diagram and explain the various process of an ideal Rankine cycle. Also derive the expression for its efficiency in terms of enthalpy.