Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	HEAT AND MASS TRANSFER  					      	     Time : 3 hours

Subject Code:	09ME204/12ME217/ME230		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The unit for thermal resistance in SI system is ____________.

2.  	The critical radius of insulation for a spherical shell is ___________

3. 	Conduction shape factor is given by _____________.

4. 	Give one example for Transient heat conduction.

5. 	The thickness of hydrodynamic boundary layer is defined as the distance from the surface at which velocity __________ is nearly zero.

6. 	Fin effectiveness is the ratio of ___________.

7.	For the same terminal temperatures, LMTD-parallel flow will be higher compared to LMTD-counter flow. (True/False)

8.  	Mass transfer is due to ____________ gradient.

9.  	An ideal absorber of radiation is also an ideal emitter. It is known as ________ law.

10. 	If the temperature of the solid surfaces changes from 270C to 6270C, its emissive power will increase in the ratio of ___________.

PART – B  (5 x 3 = 15 MARKS)

11.  	State Fourier's law of conduction.

12. 	A disk having a diameter of 30cm and maintained at a temperature of 950C is buried at a depth of 1.0 m in a semi-infinite medium having an isothermal surface temperature of 200C and a thermal conductivity of 2.1 W/m0C. Calculate the heat loss by the disk.

13. 	Differentiate between free and forced convection.

14. 	What is fouling? What are its effects on heat exchanger performance?

15. 	Distinguish between filmwise and dropwise condensation.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Explain in brief the concept of Thermal Contact Resistance.	 					(3)

b. 	An external wall of a house may be approximated by a 10 cm layer of common brick [k=0.7 W/mK] followed by a 4 cm of gypsum plaster [k=0.48 W/mK]. What thickness of loosely packed rock-wool [k= 0.065 W/mK] should be added to reduce the heat loss through the wall by 80%? 														(12)

(OR)

17. 	A current of 200 A is passed through a stainless-steel wire [k=19 W/m0C] 3 mm in diameter. The resistivity of the steel may be taken as 70µ cm, and the length of the wire is 1m. The wire is submerged in a liquid at 1100C and experiences a convection heat-transfer coefficient of 4 kW/m2C. Calculate the centre temperature of the wire.

18. 	A steel ball [c=0.46 kJ/kg0C, k=35 W/m0C] 5.0 cm in diameter and initially at a uniform temperature of 4500C is suddenly placed in a controlled environment in which the temperature is maintained at 1000C. The convection heat-transfer coefficient is 10 W/m2.0C. Calculate the time required for the ball to attain a temperature of 1500C.

[P.T.O]

(OR)

19. 	Ice ball of 10 mm diameter at -320C are exposed to an air current at 150C with a convection heat transfer coefficient of 200 W/m2K. Determine the time when the surface layer will began to melt. Also determine the center, mid surface temperatures and heat removed. Use the following property values, density=920kg/m3, specific heat= 2040 J/kgK. Thermal conductivity =2 W/mK.

20. 	State Buckingham’s  theorem. Explain the various parameters used in forced convection. Using dimensional analysis obtain an expression for Nusselt number in terms of Reynolds and Prandtl numbers.

(OR)

21.  Air at 200C and 1 atm flows over a flat plate at 35 m/s. The plate is 75 cm long and is 	maintained at 600C. Assuming unit depth in z direction, calculate the heat transfer from the 	plate.

22.  A hot fluid at 200 0C enters a heat exchanger at a mass flow rate of 104 kg/hr. Its specific heat is 	2000 J/kgK. It is to be cooled by another fluid entering at 250C with a mass flow rate 2500 	kg/hr and specific heat 400 J/kgK. The overall heat transfer coefficient based on outside area of 	20 m2 is 250 W/m2K. Find the exit temperature of the hot fluid when the fluids are in parallel-	flow.

(OR)

23. 	a.	Explain Fick’s law of diffusion. What is mass diffusivity? What is its dimension.   	    (3)

b.	CO2 and air experience equimolar counter diffusion in a circular tube whose length and diameter are 1 m and 50 mm, respectively. The system is at a total pressure of 1 atm and a temperature of 250C. The ends of the tube are connected to large chambers in which the species concentrations are maintained at fixed values. The partial pressure of CO2 at one end is 190 mm of Hg while at the other end is 95 mm of Hg. Estimate the mass transfer rate of CO2 and air through the tube.     											  (12)

24.	 A vertical tube of 50 mm outside diameter and 2 m long is exposed to steam at atmospheric pressure.  The outer surface of the tube is maintained at a temperature of 84oC by circulating cold water through the tube.  Calculate the (a) the rate of heat transfer to the coolant and (b) the rate of condensation of steam.

(OR)

25. 	A thin aluminum sheet with an emissivity of 0.1 m on both sides is placed between two very large parallel plates that are maintained at uniform temperatures T1=800 K and T2=500 K and have emissivities 1=0.2 and 1=0.7 respectively. Determine the net rate of radiation heat transfer between the two plates per unit surface area of the plates and compare the result to that without the shield.