Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MECHANICS OF MACHINES - I					      	     Time : 3 hours

Subject Code:	09ME205/12ME222/ME207/ME254 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term Mechanism.

2.	What is meant by kinematic chain?

3.	Write the equation for frictional torque transmitted by a single plate clutch (considering uniform wear).

4.	Due to slip of the belt, the velocity ratio of the belt drive __________.

5.	Define offset follower.

6.	For high speed engines, the cam follower should move with __________.

7.	The radial distance of a tooth from the pitch circle to the bottom of the tooth, is called __________.

8.	When the axes of first and last gear are co-axial, then gear train is known as __________.

9.	Give an example for spring controlled governor.

10.	A governor is said to be __________ , if the speed of the engine fluctuates continuously above and below the mean speed.

PART – B  (5 x 3 = 15 MARKS)

11.	Define “Lower Pair”.

12.	List few applications of clutches.

13.	Mention any three important applications of cams.

14.	List out the advantages of gear drive.

15.	What do you mean by ‘Isochronous Governors’?

PART – C  (5 x 15 = 75 MARKS)

16. 	Sketch and explain the various inversions of four bar chain.

(OR)

17.	In a four bar chain ABCD, AD is fixed and is 150 mm long. The crank AB is 40 mm long and rotates at 120 r.p.m. clockwise, while the link CD = 80 mm oscillates about D. BC and AD are of equal length. Find the angular velocity of link CD when angle BAD = 60°.

18.	Two pulleys, one 450 mm diameter and the other 200 mm diameter are on parallel shafts 1.95m apart and are connected by a crossed belt. Find the length of the belt required and the angle of contact between the belt and each pulley. What power can be transmitted by the belt when the larger pulley rotates at 200 rev/min, if the maximum permissible tension in the belt is 1 kN, and the coefficient of friction between the belt and pulley is 0.25?

(OR)

19.	A single plate clutch, with both sides effective, has outer and inner diameters 300 mm and 200 mm respectively. The maximum intensity of pressure at any point in the contact surface is not to exceed 0.1 N/mm2. If the coefficient of friction is 0.3, determine the power transmitted by a clutch at a speed 2500 r.p.m.

[P.T.O]

20.	Explain with sketches the different types of cams and followers.

(OR)

21.	A cam is to give the following motion to a knife-edged follower : 1. Outstroke during 60° of cam rotation ; 2. Dwell for the next 30° of cam rotation ; 3. Return stroke during next 60° of cam rotation, and 4. Dwell for the remaining 210° of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft.

22.	A pinion having 30 teeth drives a gear having 80 teeth. The profile of the gears is involute with 20° pressure angle, 12 mm module and 10 mm addendum. Find the length of path of contact, arc of contact and the contact ratio.

(OR)

23.	In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth respectively. If the arm rotates at 150 r.p.m. in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of gear B. If the gear A instead of being fixed, makes 300 r.p.m. in the clockwise direction, what will be the speed of gear B ?

24.	A Porter governor has equal arms each 250 mm long and pivoted on the axis of rotation. Each ball has a mass of 5 kg and the mass of the central load on the sleeve is 25 kg. The radius of rotation of the ball is 150 mm when the governor begins to lift and 200 mm when the governor is at maximum speed. Find the minimum and maximum speeds and range of speed of the governor.

(OR)

25.	An aeroplane makes a complete half circle of 50 metres radius, towards left, when flying at 200 km per hr. The rotary engine and the propeller of the plane has a mass of 400 kg and a radius of gyration of 0.3 m. The engine rotates at 2400 r.p.m. clockwise when viewed from the rear. Find the gyroscopic couple on the aircraft and state its effect on it.