Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DESIGN OF  TRANSMISSION SYSTEMS / DESIGN OF MECHANICAL TRANSMISSION SYSTEMS													      	     												     Time : 3 hours

Subject Code:	09ME208 / 12ME224 / ME231					       Maximum Marks: 100

#### Answer ALL questions

(Use of approved data books is permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	Name the main components of a roller bearing.

2.	Specify the purpose of crowning a flat pulley.

3.	What is meant by a 6 x 19 rope?

4.	What is a herringbone gear?

5.	What is a miter gear?

6.	What is a ratchet and pawl mechanism?

7.	Define the term progression ratio of a gearbox.

8.	State the purpose of using a gear box.

9.	When does a brake become self-locking?

10.	The type of contact between a cam and a follower is _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the bearing pressure of the journal bearing for the following data: Load on the journal is 12 kN; length of the journal is 150 mm; Diameter of the journal is 80 mm; Weight of the turbine with the shaft is 40 kN and the shaft rotates at 1500 rpm.

12.	In what ways helical gears are different from the spur gears?

13.	Find the face width (b) of the ratchet teeth assuming the following data. Torque is 500 Nm; Number of teeth is 18; ψ is 2.5; Allowable bending stress is 30 N/mm2.

14.	What are the possible arrangements of structural formula for a 16 speed gear box?

15.	Define the terms: (a) Base circle (b) Prime circle (c) Pressure angle.

PART – C  (5 x 15 = 75 MARKS)

16.	For a 6307 ball bearing, the load varies as follows.

| Radial Load (N) | Axial Load (N) | Cycle time ratio | Speed in rpm |
| --- | --- | --- | --- |
| 6000 | 3000 | 0.5 | 400 |
| 7500 | --- | 0.3 | 650 |
| 4000 | 1000 | 0.2 | 900 |

The loads are steady. Find the expected average life of the bearing.

(OR)

17.	A motor driven blower is to run at 650 rpm. The electric motor transmits 7.5 kW at 180 rpm. Design the V belt drive.

18.	Design a chain drive to activate a compressor from a 10 kW motor at 960 rpm. The compressor rpm is 350 rpm. The minimum centre distance is 500 mm. Compressor is to work for 8 hours/day.

(OR)

[P.T.O]

19.	Design a pair of spur gears to transmit 20 kW at a pinion speed of 1400 rpm. The transmission ratio is 4. Assume the material of the pinion to be 15Ni2Cr1Mo15 and that of the gear to be C45.

20.	The input to worm gear is 18 kW and 600 rpm. Speed ratio is 20. The worm is made of hardened steel and the wheel is made of chilled Phosphor bronze. The number of starts on the worm is to be considered as 3.

(OR)

21.	a.	Determine the geometric dimensions of a six station Geneva wheel for the driving crank

radius of 100mm.

b.	Also find the instantaneous angular velocity and angular acceleration of the geneva wheel

when the crank angle ϕ = 20º; crank radius is 88 mm; centre distance is 125 mm; speed of

crank is 100 rpm.

22.	Spindle of a drilling machine runs at 12 different speeds in the range of 100 rpm to 355 rpm. Design a three stage gear box. Sketch the layout of the gear box and also the ray diagram. The gear box receives 5 kW from an electric motor running at 360 rpm.

(OR)

23.	A gear box is to be designed with the following specifications. Power = 14.72 kW; Number of speeds = 18; Minimum speed = 16 rpm; Step ratio = 1.25; Motor speed = 1400 rpm. The 18 speeds are obtained as 2 X 3 X 3. Sketch the layout of the gear box and the speed diagram.

24.	A 50 kg wheel, 0.5 m in diameter, turning at 150 rpm in stationary bearings is brought to rest by pressing a brake shoe radially against the rim with a force of 100 N. If the radius of gyration of wheel is 0.2 m, how many revolutions will the wheel make before coming to rest? Assume that the coefficient of friction between the shoe and the rim has a value of 0.25.

(OR)

25.	A multi plate clutch with both sides effective transmits 30 kW at 360 rpm. Inner and outer radii of clutch discs are 100 mm and 200 mm respectively. The coefficient of friction is 0.25. An axial load of 600 N is applied. Assuming uniform wear conditions, find the number of discs required and the maximum intensity of pressure developed.