Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :FLUID POWER CONTROL	/ FLUID POWER CONTROL  ENGINEERING						      	     												Time : 3 hours

Subject Code:	09ME225/12ME205/ME237		 				        Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Fluid power is also called as __________.

2.	Name the basic components required in Hydraulic System.

3.	Draw the symbols for 5/2 direction control valve-pilot operated and spring return.

4.	What is the function of a sequence valve?

5.	What are accumulators?

6.	Define mechanical efficiency of a hydraulic motor.

7.	Write the function of intensifier?

8.	Define automation.

9.	Name one application of mechanical – hydraulic servo system.

10.	Give some applications of pneumatics in CNC machining centers.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the need for automation.

12.	List various types of pumps.

13.	Distinguish between Non-return valve and relief valves.

14.	What are the fluidic elements?

15.	Draw the regenerative circuit and write its application.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the working of vane pump.                      									    (6)

b.	Find the actual delivery gear pump with the following specifications.

Outside diameter of gear pump   =  80mm

Inside   diameter of gear pump   =  60mm

Gear width                                  =  20mm

Speed of the pump                      =  1600 rpm

Volumetric efficiency                 = 88%                                          					    (9)

(OR)

17.	Discuss the merits, demerits of pneumatic and hydraulic controls and compare it with other drive system.

18.	Draw the sectional view of a Hydraulic Double acting Cylinder with cushioning arrangement and explain its operation in detail.

(OR)

19.	Explain the working of any two type of an accumulator with neat sketch.

20.	Explain meter-in and meter-out circuit and state where they are used.

(OR)

21.	Explain hydraulic sequencing circuit, with neat diagram.

22.	Explain with neat sketch the principle of operation of different flow control valves.

(OR)

[P.T.O]

23.	a.	What is a counter balance valve?  Explain its significance.							    (5)

b.	Explain with neat sketch the construction and operation of a pressure relief valve.	  (10)

24.	Explain the hydraulic circuit of surface grinding machine table.

(OR)

25.	Explain the circuit for hydraulic press.