Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	METAL CUTTING THEORY AND PRACTICE													      	     											     Time : 3 hours

Subject Code:	09ME223								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_____________type of chip results in better surface finish.

2.	What is specific cutting energy?

3.	Name any two multipoint cutting tools.

4.	What is the use of machine tool dynamometers?

5.	Name the locations where heat is produced in an orthogonal cutting operation.

6.	Write the main functions of cutting fluid.

7.	What is meant by hot hardness?

8.	Write Taylor’s tool life equation.

9.	Name the type of tool wear occurs when machining ductile material.

10.	What are the effects of chatter on workpiece?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the conditions that would promote built up edge formation?

12.	Show the main forces generated during turning operations with a neat sketch.

13.	What is hot machining? Give examples.

14.	What are the limitations of carbon steel tools?

15.	Suggest few methods to eliminate machine tool chatter.

PART – C  (5 x 15 = 75 MARKS)

16.	With neat sketch, explain the types of chip formed in metal cutting along with the conditions for their formation.

(OR)

17.	Following data were collected during orthogonal turning process: Depth of cut = 2 mm, chip thickness=0.62 mm, feed =0.2 mm/rev, rake angle = 15o, average coefficient of friction 0.7 and tangential cutting force =1600 N. Calculate: a) chip thickness ratio, b) shear plane angle, c) friction angle d) shear stain involved in the deformation process and e) specific cutting energy.

18.	Explain in detail the geometry of a single point cutting tool with neat sketch.

(OR)

19.	Explain the principle of measuring cutting force using strain gauge dynamometer in turning process.

20.	Explain the tool work thermocouple method of measuring cutting temperature.

(OR)

21.	a.	State the essential properties of a cutting fluid.					 				    (5)

b.	Discuss various methods of applying cutting fluid for effectively reducing cutting temperature. 																	  (10)

[P.T.O]

22.	Discuss the main characteristics of the following tool materials:

a.	HSS, 	b.	Cemented Carbide, 	c.	Ceramics.

(OR)

23.	What is meant by machinability? Discuss the factors that affect materials machinability.

24.	With neat sketch, explain the various forms of cutting tool wear.

(OR)

25.	Discuss the effects of various factors on machine tool chatter.