Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DC MACHINES AND TRANSFORMERS			      	     Time : 3 hours

Subject Code:	11EE207/12EE207/EE250						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The component which is used to convert alternating emf to direct emf in the output of a 		DC 	Generator is __________.

2.	If a Lap wound DC Machine has Z conductors, A parallel paths and P poles, number of 		conductors per path will be __________.

3.	Give the expression for emf equation of a wave wound DC Generator.

4.	Laminating the core structure is to reduce the eddy current losses in a DC Machine. True/False

5.	Write the Torque equation of a DC Motor.

6.	Why the emf in DC Motor is called as Back Emf?

7.	DC __________ motors are preferred for high torque applications.

8.	The total current taken by an actual transformer on no-load is given by Io =	_______.

9.	What are the losses in a transformer?

10.	Why Transformer is rated in KVA instead of KW?

PART – B  (5 x 3 = 15 MARKS)

11. 	A 6 pole, lap wound DC Generator has 51 Slots and each slot containing 18 conductors. 	The useful flux is 35 milli webers. Determine the generated emf in the armature, if it is drive at a speed of 750 RPM.

12.	How to reduce magnetic losses in a DC Machine?

13.	What are the speed control methods of DC Motor?

14.	List out the applications of DC Series Motor.

15.	Explain Transformation ratio of a Transformer.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Draw and explain the electromechanical energy conversion system. 				    (7)

b.	A separately excited DC Generator running at 1000 rpm is supplied 110A at 220V to a 			resistive load. If the load resistance remains constant, what will be the load current if the 			speed is reduced to 800 rpm? Armature resistance is 0.02 Ω. Field current is unaltered. 			Assume a voltage drop of 1V per brush. Ignore the effect of armature reaction. 		    (8)

(OR)

17.	Describe the working principle DC Generator and illustrate its constructional details with 		neat diagrams.

18. 	a.	A 110 KW belt driven DC Shunt generator runs at 375 RPM on 220V Bus bar continues 		to run as a DC Motor when the belt breaks taking a Power of 5.5 KW. Obtain its speed as 	a DC Motor, if the armature and field resistances are 0.025 Ω and 110 Ω respectively. 	Assume BCD as 1V per brush. 													  (10)      b.	Sketch the O.C.C. of DC Generator.											    (5)

(OR)

[P.T.O]

19. 	a.	Compare Lap and wave armature windings of a DC Machine.						    (5)

b.	Derive the emf equation of a DC Generator.										    (5)

c.	Describe armature reaction.													    (5)

20.	A 110 KW belt driven DC Shunt generator runs at 375 RPM on 220V Bus bar continues to run as a DC Motor when the belt breaks taking a Power of 5.5 KW. Obtain its speed as a DC Motor, if the armature and field resistances are 0.025 Ω and 110 Ω respectively. 	Assume BCD as 1V per brush.

(OR)

21. 	a.	Derive the Torque Equation of a DC Motor.										    (8)

b.	Discuss the various characteristics of DC Shunt and Series Motor.					    (7)

22.	Derive the equivalent circuit of ideal single phase transformer in steps with reference to the primary side.

(OR)

23.	 a.	A 5 kV/300 V 1- phase transformer draws a no-load current of 0.5A and absorbs 700W 			on no-load. Find the magnetizing current (Im) and iron loss current (Iw or Il)			    (5)

b.	Sketch and explain the no load phasor diagram of a single phase transformer.   		  (10)

24.	Explain the principle of operation of an auto-transformer.

(OR)

25. 	a.	Explain the working principle of a transformer.			                        			    (5)

b.	List out the applications of transformer.											    (5)

c.  	Obtain the EMF equation of single phase transformer.							    (5)