Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	11MT210 / 12MT211 / MT227	 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define ‘adjacent squares’ in a k-map.

2.	What is a parity bit?

3.	Draw the truth-table of a XOR gate.

4.	What are the applications of multiplexer?

5.	What is a sequential circuit?

6.	Draw the excitation table of J-K flip-flop.

7.	What is edge triggering?

8.	What is an excitation table?

9.	Which is the fastest logic family?

10.	What is tri-state output?

PART – B  (5 x 3 = 15 MARKS)

11.	State De Morgans theorem. Find the complement of F=xy’ +x’y.

12.	Implement Y = (0, 2, 5, 7) using logic gates.

13.	Derive the characteristic equation of SR flip-flop.

14.	What is an essential hazard?

15.	Distinguish PAL and PLA.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the minimal sum of product and product of sum expressions for the following function using K-Map method. Implement the reduced Boolean expression using logic gates

f (w,x,y,z) = (0,2,3,5,7,8,13) +  d(1,6,12)

(OR)

17.	a.	Perform the following conversions												(3x2)

i.	(153.513)10 to octal

ii.	(10110001101011.11110010)2 to hexadecimal

iii.	(9673.124)8 to binary

b.	Given the two binary numbers X = 1010100 and Y=1000011, perform the subtraction X-Y

using 1’s complement and 2’s complement method.								    (9)

18.	Design a Full adder using (a) two half-adders and (b) NAND gates.

(OR)

19.	Design a 4 – bit Binary to Gray code converter and explain its operation.

20.	With a brief note on need for master-slave flip-flops, explain a master slave JK flip-flop in detail.

(OR)

21.	Explain the operation of a 4 bit Johnson counter with a timing diagram.

22.	Explain the Mealy and Moore Models of sequential circuit.

(OR)

23.	Design a sequence detector that produces an output ‘1’ whenever the non-overlapping sequence 1011 is detected.

24.	Compare the characteristics of TTL and CMOS Logic families.

(OR)

25.	Design a BCD to Excess-3 code converter using PLA.