Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	THERMODYNAMICS								      	     Time : 3 hours

Subject Code:	12AE202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define thermodynamic equilibrium in a system.

2.	State the general principle of the conservation of energy.

3.	Define thermal energy reservoir.

4.	Name 2 examples of high grade energy.

5.	What is the use of a separating and throttling calorimeter?

6.	Define reduced pressure.

7.	What is meant by metastable equilibrium?

8.	When an ideal gas is made to undergo a throttling process, there is no change in temperature. True or False?

9.	Write the de Broglie equation.

10.	State Pauli’s exclusion principle.

PART – B  (5 x 3 = 15 MARKS)

11.	What is quasi-static process and state its characteristic feature.

12.	Draw a cyclic heat pump and write the formula for COP of heat pump.

13.	Draw a p-V-T surface of a pure substance and indicate the regions.

14.	State and explain Gibbs phase change rule for a non-reactive system.

15.	Write a note on the wave-particle duality of light.

PART – C  (5 x 15 = 75 MARKS)

16.	In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and the velocity at the inlet are 0.37 m3/kg, 600 kPa and 16 m/s respectively. The inlet is 32 m above the floor and the discharge pipe is at the floor level. The discharge conditions are 0.62 m3/kg, 100 kPa and 270 m/s. The total heat loss between the inlet and discharge is 9 kJ/kg of fluid. Calculate the decrease in specific internal energy of the fluid while it flows through this apparatus.

(OR)

17.	Air at a temperature of 15C passes through a heat exchanger at a velocity of 30 m/s where its temperature is raised to 800C. It then enters a turbine with the same velocity of 30 m/s and expands until the temperature falls to 650C. On leaving the turbine, the air is taken at a velocity of 60 m/s to a nozzle, where it expands until the temperature has fallen to 500C. If the air flow rate is 2 kg/s, calculate (a) the rate of heat transfer to the air in the heat exchanger       (b) the power output from the turbine assuming no heat loss and (c) the velocity at the exit from the nozzle, assuming no heat loss. The enthalpy of air is h=Cpt, where Cp is the specific heat capacity equal to 1.005 kJ/kgK and t is the temperature.

18.	Explain the process and working of a Carnot cycle with the help of a line sketch and p-V diagram.

(OR)

[P.T.O]

19.	A mass of 1000 kg of fish initially at 1 bar, 300 K is to be cooled to -20C. The freezing point of fish is -2.2C and the specific heat of fish below and above the freezing point are 1.7 and 3.2 kJ/kg K respectively. The latent heat of fusion for fish is 235 kJ/kg. Calculate the energy produced in the chilling plant. Take To = 300 K and Po = 1 bar.

20.	Steam initially at 1.5 Mpa, 300C expands reversibly and adiabatically in a steam turbine to 40C. Determine the ideal work output of the turbine per kg of steam.

(OR)

21.	Explain the principle and working of a throttling calorimeter with a neat sketch.

22.	Explain the Joule-Kelvin effect with a sketch and show the isenthalpic and inversion curves.

(OR)

23.	Derive the Tds equations and prove that

24.	a.	Write a short note on Heisenberg’s uncertainty principle.							    (5)

b.	Consider a cubical box of edge 10 cm, containing gaseous helium at 300 K. Evaluate the energy x and its corresponding nx.												  (10)

(OR)

25.	a.	Write a brief note on Maxwell-Boltzmann statistics.								  (10)

b.	Calculate the number of ways of arranging six indistinguishable particles in either distinguishable boxes with no more than one particle per box.						    (5)