Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	STRENGTH OF AEROSPACE MATERIALS		      	     Time : 3 hours

Subject Code:	12AE203								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define stress.

2.	State Hooke’s law.

3.	A cantilever beam of span ‘l’, carries a point load ‘W’ at the free end of the beam. The maximum bending moment is given by _________.

4.	Draw the bending moment diagram for a cantilever  beam of  span L subjected to a point load at the free end.

5.	Define neutral plane.

6.	The section modulus Z of a hollow rectangular section  is __________.

7.	Define slenderness ratio.

8.	The deflection at the free end of a cantilever beam carrying a point load W at the free end is _________.

9.	Define the complementary shear stresses.

10.	Name any two theories of failure.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the terms modulus of elasticity and Poisson’s ratio.

12.	A cantilever of length 2 m carries a point load of 5 kN at 1m from the free end.  Draw SFD for the cantilever.

13.	Write the assumptions made in the simple torsion theory.

14.	Define the terms: columns and strut.

15.	Explain Total strain energy per unit volume theory.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the relationship between the Young’s modulus of elasticity  (E)  and Shear modulus of rigidity (G).

(OR)

17.	a.	Explain clearly the different types of stresses and strains.							  (10)

b.	Draw the stress and strain curve for Mild steel.									    (5)

[P.T.O]

18.	Draw the shear force and bending moment diagram for the given beam.

(OR)

19.	Draw the shear force and bending moment diagram for the given beam.

20.	Calculate the stress produce due to bending at a point A in a uniform rectangular beam 200 mm deep and 100 mm wide, simply supported at each end over a span of 3m carrying a uniformly distributed load of 15,000 N/m.

(OR)

21.	A stepped solid circular shaft is built in its ends and subject to an externally applied torque To at the shoulder as shown in the figure. Determine the angle of rotation qo of the shoulder section where To is applied. Assume modulus of rigidity for both the portion is same.

22.	Derive an expression for Euler’s crippling load for a column when one end of the column is fixed and the other end is free.

(OR)

23.	A  cantilever beam  is subjected to a point load W at free end. Find

a.	The slope at the free end and

b.	Maximum deflection.

24.	Determine the expression for strain energy of the prismatic beam AB for the loading as shown in figure below. Take into account only the effect of normal stress due to bending.

(OR)

25.	Define and explain the following theories of failure:

a.	Maximum principal stress theory.

b.	Maximum shear stress theory.