Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	AERODYNAMICS									      	     Time : 3 hours

Subject Code:	11AE205/ 12AE204								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the sources involved in aerodynamics?

2.	What are the differences between continuum flow and free molecule flow?

3.	What is the physical principle behind energy equation?

4.	Define Path line.

5.	What is a free vortex flow?

6.	What is potential and stream function for source flow?

7.	In order to generate lift, the upper surface of an airfoil must be more strongly curved than the lower surface. (Yes or No)

8.	What is Kevin's circulation theorem?

9.	State Helmholtz theorem.

10.	Define Geometric twist and Aerodynamic twist.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the different speed types of flows used in identifying Mach number?

12.	For the velocity field   u = y/(x2 + y2) and   v = - x/(x2 + y2) Calculate the circulation 	round 	a circular path of radius 5m. Assume that u and v are in units of m/s.

13.	What is D’Alembert Paradox and Magnus effect?

14.	Explain the nomenclature of NACA 2412 and NACA 23012 Airfoil.

15.	How do wing tip vortices affect an airplane?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Briefly discuss the divergence and curl of a vector field and their physical meaning.	    (8)

b.	Explain in detail Line, surface and volume integrals and the relationships between them.(7)

(OR)

17.	Explain in detail about the Aerodynamic forces and moments with neat sketches.

18.	The following cases represent the two velocity components; determine the third component of 	velocity such that they satisfy the continuity equation:

(i) u = x2 + y2 + z2 ; v = xy2 - yz2 + xy; (ii) v = 2y2 ; w = 2xyz.   				    		    (8)

(ii) In a two dimensional incompressible flow the fluid velocity components are given by

u= x-4y,   v= -y-4x. Show that the velocity potential exists and determines its form as well as stream function. 												    					    (7)

(OR)

19.	Derive the continuity equation for unsteady three dimensional flows in partial differential form using a control volume approach.

20.	Consider the lifting flow over a circular cylinder with a diameter of 0.5 m. The free stream velocity is 25 m/s, and the maximum velocity on the surface of the cylinder is 75 m/s. The free stream conditions are those for a standard altitude of 3 km (ρ= 0.90926 kg/m3). Calculate the lift per unit span on the cylinder.

[P.T.O]

(OR)

21.	Consider a lifting flow over a cylinder. The lift co-efficient Cℓ = 5, calculate the peak pressure 	coefficient and calculate the location of the stagnation points and the points on the cylinder 	where the pressure equals the free stream static pressure.

22.	a.	Explain in detail Airfoil characteristics and nomenclature.		  		    			    (8)

b.	Consider an NACA 23012 airfoil. The mean camber line for this airfoil is given by

zc=2.6595xc3−0.6075xc2+0.1147xc	for 0≤xc≤0.2025

and	zc=0.022081−xc					       		for 0.2025≤xc≤1.0

Calculate: (i) The angle of attack at zero lift,	  (ii) the lift coefficient when α = 4 º		    (7)

(OR)

23.	Derive the fundamental equation of thin airfoil theory and derive the expression for lift coefficient for a thin symmetrical airfoil using thin airfoil theory.

24   	Write short notes on: Downwash and Induced drag.

(OR)

25.	Derive the fundamental equation of Prandtl’s lifting line theory. Discuss the limitations of lifting line theory.