Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	THERMAL ENGINEERING						      	     Time : 3 hours

Subject Code:	12AE207						 				       Maximum Marks: 100

#### Answer ALL questions

#### (Use of standard steam Tables, Refrigeration charts and Mollier diagram are permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	State Boyle’s law.

2.	Define law of corresponding states.

3.	In a ____________ nozzle, the cross-section first tapers to a smaller section and then diverges to a large diameter.

4.	Write the classification of steam turbines according to the direction of steam flow.

5.	Write the 4 processes in a Carnot cycle.

6.	_____________ is one of the favorable fluids used as working medium in a binary vapour cycle.

7.	Presence of Sulphur is considered desirable. True or False.

8.	Define lower calorific value of a fuel.

9.	Define COP of a refrigeration system.

10.	Define draft in an air-conditioning system.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw a p-V-T diagram of ideal gas and name the regions.

12.	State the reasons for the reduction in velocity in a steam nozzle.

13.	State 3 methods used for improving the efficiency of Rankine cycle.

14.	Write Dulong’s formula for calculation of calorific value.

15.	State the merits and demerits of air refrigeration system.

PART – C  (5 x 15 = 75 MARKS)

16.	1 kg of air at a pressure of 8 bar and a temperature of 100C undergoes a reversible polytropic process following the law pv1.2=constant. If the final pressure is 1.8 bar, determine a) final specific volume b) final temperature and c) increase in entropy

Assume R = 0.287 kJ/kg K and  = 1.4.

(OR)

17.	Explain the process of adiabatic mixing of perfect gases with a sketch and derive the expression for the temperature of a mixture.

18.	Dry saturated steam enters a steam nozzle at a pressure of 15 bar and is discharged at a pressure of 2.0 bar. If the dryness fraction of the discharge steam is 0.96, what will be the final velocity of the steam ? Neglect the initial velocity of steam. If 10% of the heat drop is lost in friction, find the percentage reduction in the final velocity.

(OR)

19.	Explain the variation of pressure and velocity of steam in a velocity compounded steam turbine with a sketch.

[P.T.O]

20.	An engine of 250 mm bore and 375 mm stroke works on Otto cycle. The clearance volume is 0.00263 m3. The initial pressure and temperature are 1 bar and 50C. If the maximum pressure is limited to 25 bar, find the following:

a.	Air standard efficiency.

b.	Mean effective pressure of the cycle. Assume ideal conditions.

(OR)

21.	Explain the working of a reheat cycle with a line diagram and T-S diagram.

22.	With a neat sketch explain the working principle of Orsat apparatus used for flue-gas analysis.

(OR)

23.	Explain the determination of calorific value of a fuel using Bomb calorimeter with a neat sketch.

24.	Explain the principle and working of a vapour absorption refrigeration system with a line diagram.

(OR)

25.	Briefly explain the various arrangements of supply duct and air distribution in Air-conditioning systems.