Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	AIRCRAFT STRUCTURES							      	     Time : 3 hours

Subject Code:	12AE212						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are conditions of equilibrium used in the method of joints?

2.	What do you mean by a cantilever beam ?

3.	Major structural member of an aircraft fuselage are ______and ______.

4.	Define Neutral plane.

5.	What is shear center?

6.	Determine the Ixy  of the thin walled section shown in the figure.

7.	What is slenderness ratio?

8.	What is thin plate and how is it used for basic part of an aero plane structure?

9.	Name any two Matrix Materials.

10.	What are the limitations of composite materials?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between statically determinate and inderminate structure.

12.	What are the structural components of the wing?

13.	Define Shear flow.

14.	How slenderness ratio affects the strength of thin walled column?

15.	What is an impact of reinforcement in composite material?

PART – C  (5 x 15 = 75 MARKS)

16.	Compute the force in all members of the Truss shown in figure.

[P.T.O]

(OR)

17.	What is the Clapeyron’s theorm of three moments? Derive an expression for Clapeyron’s theorm of three moments.

18.	With help of neat sketch explain the wing construction fuselage construction of an airplane.

(OR)

19.	The fuselage section shown in fig. is subjected to a bending moment of 100 KN m applied in the vertical plane of symmetry. If the section is completely idealized into a combination of direct stress-carrying booms and shear stress only carrying panels, determine the direct stress in each boom.

20.	A beam having the cross section shown in figure is subjected to a bending moment of 1500Nm in a vertical plane. Calculate the maximum direct stress due to bending and also state the point at which it acts.

(OR)

21.	The cross section of a beam has the dimension shown in fig. If the beam is subjected to a negative bending moment of 100 kN m applied in a vertical plane determine the distribution of direct stress through the depth of the section.

22.	A circular flate plate of diameter 120 mm and thickness 10 mm is constructed from steel with

E = 208 GN/m2 and υ = 0.3. The plate is subjected to a uniform pressure of 5 MN/m2 on one side only. If the plate is clamped at the edge determine.

a.	The maximum deflection.

b.	The position and magnitude of the maximum radial stress.

c.	What percentage change in the results will be obtained if the edge condition is such that the plate can be assumed to be freely supported?

(OR)

23.	Write short notes on

a.	Needham Method   and														    (7)

b.	Gerard method.																    (8)

24.	a.	What is composite material?													    (4)

b.	Give advantages of  carbon –fiber-reinforced plastics (CFRP) over  glass-reinforced plastic (GRP).																		    (8)

c.	Give application of CFRP. 													    (3)

(OR)

25.	Write short notes on

a.   Classification of Composite														    (5)

- Advantages  and limitations of Composites. 										  (10)