Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	APPLIED HYDRAULICS AND FLUID MACHINES		      Time : 3 hours

Subject Code:	09CE216/12CE209/CE246						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define open channel flow.

2.	Define hydraulic jump.

3.	Define pressure drag.

4.	What is boundary layer?

5.	Define Impact of a jet.

6.	What is jet propulsion?

7.	What is a turbine?

8.	Define specific speed of turbine.

9.	What are the uses of multistage pumps?

10.	What is suction head?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the terms: 	a.	Rapidly varied flow 		b.	Gradually varied flow.

12.	What is meant by separation of a boundary layer? What is the effect of pressure gradient on boundary layer separation?

13.	Differentiate between the force exerted by a jet of water on a fixed vertical plate and moving vertical plate.

14.	Distinguish between Reaction turbine and Impulse turbine.

15.	Define ‘cavitation’

PART – C  (5 x 15 = 75 MARKS)

16.	A trapezoidal channel has side slopes of 1 horizontal to 2 vertical and the slope of the bed is 1 in 2000. The area of the section is 42m2. Find the dimensions of the section if it is most economical. Determine the discharge of the most economical section if C=60.

(OR)

17.	A hydraulic jump forms at the downstream end of spillway carrying 17.93 m3/s discharge.  If the depth before jump is 0.80 m, determine the depth after the jump and the energy loss.

18.	Find the ratios of displacement thickness to momentum thickness and momentum thickness to energy thickness for the velocity distribution in the boundary layer given by

u/U = 2(y/δ) – (y/δ)2 where u = Velocity in boundary layer at distance y, U = Free-stream velocity,  δ = Boundary layer thickness.

(OR)

19.	Calculate the diameter of a parachute to be used for dropping an object of mass 100 kg so that the maximum terminal velocity of dropping is 5m/s. The drag coefficient for the parachute which may be treated as hemispherical is 1.3.  The density of air is 1.216 kg/m3.

[P.T.O]

20.	A jet of water of diameter 50 mm strikes a fixed plate in such a way that the angle between the plate and the jet is 30 degree.  The force exerted in the direction of the jet is 1471.5 N.  Find the rate of flow of water.

(OR)

21.	A jet of water of diameter 100 mm strikes a curved plate at its centre with a velocity of 15 m/s. The curved plate is moving with a velocity of 7m/s in the direction of the jet. The jet is deflected through an angle of 150°. Assuming the plate smooth find:

a.	force exerted on the plate in the direction of the jet.

b.	power of the jet.

c.	efficiency.

22.	Explain the working of a Kaplan turbine with a neat sketch.

(OR)

23.	The following data relates to a pelton wheel, Head = 72 m, speed of wheel = 340 R.P.M, shaft power of wheel 115 KW, speed ratio = 0.45, co efficient of velocity = 0.98, overall efficiency 85%. Design the pelton wheel.

24.	With a neat sketch, explain the principle and working of a centrifugal pump.

(OR)

25.	A centrifugal pump is to discharge 0.118 m3/s at a speed of 1450 rpm against a head of 25 m.  The impeller diameter is 250 mm, its width at outlet is 50 mm and manometric efficiency is 75%.  Determine the vane angle at the outer periphery of the impeller.