Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	REINFORCED CONCRETE STRUCTURES / REINFORCED CONCRETE STRUCTURES - I			      	     			     Time : 3 hours

Subject Code:	09CE210/12CE217/CE249						      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.    	What are the design requirements considered in RCC design?

2. 	What is the mix proportion of M20 concrete?

3.	What is a lintel beam?

4.    	Define flight.

5.    	Give the expression for the moment of resistance of singly reinforced beam in LSM.

6.    What is a continuous beam?

7.    	List the different types of footings with a neat sketch.

8.    	Define slenderness ratio.

9.    	What is the formula used for checking cracking tensile stress in a water tank?

10.  	Name any two types of retaining walls.

PART – B  (5 x 3 = 15 MARKS)

11. 	Draw neutral axis for a under-reinforced, over-reinforced and balanced sections beam.

12. 	Classify the types of staircases with suitable diagrams.

13. 	As per IS: 456-2000, how to calculate shear for rectangular beams?

14. 	List the basic assumptions for the design of compression members.

15.  Explain with sketches, different types of water tanks.

PART – C  (5 x 15 = 75 MARKS)

16. 	Differentiate the different methods of design like working stress method, ultimate load method and the limit state method.

(OR)

17. 	A reinforced concrete beam 250 mm x 500 mm overall depth is reinforced with 3 nos. of 16mm dia. steel bars in the tension zone. The nominal cover to the reinforcement is 25mm. Determine the position of actual neutral axis, if m=18.7. If the maximum compressive stress in concrete is 4.8 N/mm2, what will be the tensile stress in steel?

18. 	Design a simply supported R.C roof slab for a watchman cabin of clear size 2m x 3m. The thickness of walls alround is 200mm. Access are not provided to the roof. The corners of the slab are not held down. Weight of the weathering course will be 1kN/m2. Concrete grade M15 and mild steel grade of Fe.250 are to be used.

(OR)

19. 	Design a dog-legged staircase between two intermediate R.C floors of a multistoried building using concrete grade M20 and Fe415 steel. The vertical height between floors is 2.7 meters. The clear size of the staircase room is 5.1m x 2.5m. Imposed u.d.l on stairs is 3000N/m2. Width of supporting walls 300mm. The flight slab and landing span longitudinally. Weight of brick steps may be taken as 2000N/m3.

[P.T.O]

20. 	A simply supported rectangular beam of effective span 6.3m is subjected to a total u.d.l of 13.5kN/m (inclusive of seil weight) throughout its length. Design the mid span section for the limit state of collapse in flexure using M20 grade of concrete and Fe415 grade of steel. The breadth of beam is 300mm. Assume a partial safety factor of 1.5 for the load.

(OR)

21. 	A simply supported rectangular beam of 300 mm x 500mm effective size carries a total characteristic load of 80kN. The percentage of tension steel at the support section is found to be 0.628. The beam is of M20 grade concrete. Design the shear reinforcement using mild steel bars by limit state method.

22. 	Design a reinforced concrete column 300 mm x 500 mm, to carry an ultimate load of 1000 kN. Use M 20 grade concrete and Fe 250 steel. Assume it as a short column.

(OR)

23. 	Design a rectangular R.C footing of uniform thickness for an isolated rectangular R.C column of 300mm x 450mm for carrying an axial load of 800 kN respectively using M 20 concrete and Fe 415 steel. The safe bearing capacity of soil may be taken as 150 kN/m2.

24. 	Design a cantilever retaining wall to retain an earth embankment with a horizontal top 4 m above ground level. Density of earth =18 kN/m3. Angle of internal friction = 300. SBC of soil = 200 kN/m2. Coefficient of friction between soil and concrete = 0.5. Adopt M20 Grade concrete and Fe 415 HYSD bars.

(OR)

25.  	Design a circular water tank with flexible connection at base for a capacity of 4,00,000 liters. The tank rests on a firm level ground. The height of tank including a free board of 200 mm should not exceed 3.5m. The tank is open at top. Use M 20 concrete and Fe 415 steel.