Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	STRUCTURAL ANALYSIS-II/ ADVANCED STRUCTURAL ANALYSIS													      	     					     Time : 3 hours

Subject Code:	09CE218 / 12CE224 / CE255						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If the hinged end of a propped cantilever of span L settles by an amount of 8mm, then the rotation of the hinged end will be __________.

2.	What do you mean by statically indeterminate beam?

3.	Define: Carry Over Moment.

4.	What is the moment at the fixed end, when the moment ‘M’is applied do the hinged end of a prismatic propped cantilever?

5.	What is a unit matrix?

6.	What is “lack of fit”?

7.	List any two structural analysis software.

8.	A uniform propped cantilever has span L and flexural rigidity EI.  The stiffness coefficient corresponding to rotation of propped end is __________.

9.	Define: Damping.

10.	Write the characteristic equation for undamped free vibration of SDOF.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the assumptions made in the slope deflection method of analysis?

12.	Define stiffness factor with neat sketches.

13.	Will there be any difference in applying flexibility method for determinate structures and indeterminate structures? Justify your answer.

14.	The stiffness of a beam is given as4224. When the nodal forces are108, find the nodal     displacements.

15.	What is the effect of damping on structural response to dynamic loads?

PART – C  (5 x 15 = 75 MARKS)

16.	A beam ABC, 10m long, fixed at ends A and B is continuous over joint B and is loaded as shown in figure.  Using the slope deflection method, compute the end moments and plot the BMD. EI is constant for both the spans.

(OR)

[P.T.O]

17.	Analyse the frame shown in figure for moments at the ends of members. EI is constant for all the members.

18.	Analyse the continuous beam in Figure.  Find the reactions and support moments by method of moment distribution. Draw the bending moment diagram and shear force diagram.

(OR)

19.	Determine the support moments of the frame shown in the figure using moment distribution method.

20.	Analyse the continuous beam in figure by flexibility method. Draw the BMD.

(OR)

21.	Analyse the portal frame ABCD shown in figure by flexibility matrix method. EI is constant throughout.

22.	A two span continuous beam ABC is fixed at A and simply supported over the supports at B and C.  AB = 6m, BC = 4m.  Moment of inertia is constant throughout.  A uniformly distributed load of 2kN/m acts over AB and a single concentrated central load of 6kN acts on BC.  Analyse the beam by stiffness method.

(OR)

23.	Analyse the frame shown in figure by Matrix Stiffness method. Draw the BMD.

24.	Write short notes on: (a) Free Vibration; (b) Forced Vibration; and (c) Damping.

(OR)

25.	A shaft 10mm in diameter and 0.2m long connects a generator to the main engine.  If the mass moment of inertia of the generator rotor is 0.55Nmm – S2, determine the natural frequency in torsion (G =8 x106 N/cm2).