Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	SANITARY ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE226						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a combined sewer?

2.	What is meant by septic sewage?

3.	Define sewerage.

4.	Mention the constituents of wet weather flow.

5.	Classify bacteria based on temperature surveillance.

6.	Define aerobic decomposition.

7.	What is self cleansing velocity?

8.	Give the tolerance limit of TSS for safe discharge into surface water bodies.

9.	Give the principal methods of sewage disposal.

10.	What is a cess pool?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the various types of solids in wastewater.

12.	Write short notes on different types of sewer materials.

13.	Sketch the layout of a sewage treatment plant.

14.	Enumerate the factors affecting sludge digestion.

15.	Make a sketch of oxygen sag curve.

PART – C  (5 x 15 = 75 MARKS)

16.	The drainage area of one sector of a town is 12 hectares having an average coefficient of runoff of 0.42. Find the maximum runoff if the maximum rain intensity is 1 cm/hr. If the per capita demand of the town is 150 lpcd and the total population is 50,000, calculate the discharge for which a combined sewer has to be designed for the town.

(OR)

17.	Explain in detail (a) BOD 		(b) COD 		(c) nitrogen compounds.

18.	Design a circular sewer if the design discharge is 600 l/s running half full. Assume s = 0.0001 and n = 0.014.

(OR)

19.	With neat sketch explain the laying, jointing and testing of sewers.

20.	a.	Differentiate suspended and attached growth process.								    (6)

b.	With neat sketch explain the working of a trickling filter.							    (9)

(OR)

21.	a.	Explain the functioning of an oxidation pond.									    (7)

b.	Design a sedimentation tank for treating 5 MLD of wastewater for a city. Assume the 	required data.																    (8)

[P.T.O]

22.	a.	Explain the process of anaerobic sludge digestion.								    (8)

b.	Describe the process of dewatering and thickening of sludge.						    (7)

(OR)

23.	Explain the Streeter Phelps method of analysis of oxygen deficit in a stream.

24.	With neat sketch, elaborate on the systems of sanitary plumbing in a building.

(OR)

25.	Describe the functions of traps. Also enumerate the types of traps with neat sketch.