Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	ELECTRIC CIRCUIT ANALYSIS					      	     Time : 3 hours

Subject Code:	12EC202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchoff’s voltage law.

2.	A 1200 Watt hair dryer is plugged into a 120 volt circuit. Examine the current drawn by the hair dryer.

3.	Define resonant frequency.

4.	In a star-connected system, the relation between line voltage VL and Phase voltage Vph  is ____________.

5.	Define a loop.

6.	Two inductively coupled coils have self inductances L1 =50mH and L2 = 200mH. If the coefficient of coupling is 0.5, find the value of mutual inductance between the coils?

7.	Norton’s theorem is dual of ___________ theorem.

8.	Under the conditions of maximum power transfer, the efficiency is ___________.

9.	What is meant by forced response?

10.	The time constant of a series RL circuit is (a) LR   (b) L/R   (c) R/L

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the equivalent resistance between the terminals A and B.

12.	Find the resonant frequency for the ideal series RLC circuit having L = 50 mH, C = 0.01 μF and Supply voltage V = 10 V.

13.	Determine the current through each resistor in the circuit using mesh analysis.

6Ω	            4Ω

42V	   		   3Ω		10V

14.	Calculate the Thevenin’s Resistance at the terminals a and b of the circuit in Figure.

[P.T.O]

15.	A series RL circuit with R=30Ω and L=15H has a constant voltage V=60V applied at t=0 as 	shown in the circuit. Determine the current I, the voltage across resistor and the voltage across 	the inductor.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Find the currents in all resistors supplied by the battery							  (10)

b.	Determine Vx in the circuit.													    (5)

(OR)

17.	Use the technique of star-delta conversion to find the equivalent resistance of the given circuit. Each resistor R is 10Ω.

18.	a.	For the circuit shown, determine the impedance at resonant frequency, 10Hz above resonant frequency and 10Hz below resonant frequency.								  (10)

b.	Calculate the total inductance of three coupled circuit shown in the figure.			    (5)

(OR)

19.	a.	A three-phase, balanced delta-connected load of (4+j3)Ω is connected across a 400V, 3-			phase balanced supply. Determine the phase currents, line currents and power drawn by the 		load. Assume the phase sequence to be RYB.									  (12)

b.	The two wattmeter method is used to measure power in a 3-phase load. The wattmeter readings are 400W and -35W. Calculate (a) power factor  (b) reactive power.		    (3)

20.	Use mesh analysis to determine the three mesh currents in the circuit.

(OR)

21.	Use nodal analysis, to determine the voltages at each node for the circuit shown.

[P.T.O]

22.	a.	Find the value of RL for the given network below that the power is maximum? And also 	find the Max Power through load-resistance RL by using maximum power transfer 	theorem?																	  (10)

b.	Use the superposition theorem to find v in the circuit in Figure.   					    (5)

(OR)

23.	Apply Thevenin’s theorem to determine the current through 30Ω resistor in the circuit shown in Figure.

24.	A series RC circuit consists of resistor of 10Ω and capacitor of 0.1F as shown in the circuit. A constant voltage of 20V is applied to the circuit at t=0. Obtain the current equation. Determine the voltages across the resistor and the capacitor.

(OR)

25.	Derive an expression for the current of the given RC circuit.