Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	12EC203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the three co ordinate systems.

2.	Write the relation which reflects divergence theorem.

3.	Write an equation to find the force between two static charges.

4.	What is the relation between the electrostatic field intensity and the electric scalar potential.

5.	Write the mathematical relation between magnetic flux density and magnetic field intensity in free space.

6.	Write the Lorentz force equation.

7.	Write the point form of Ohm’s law.

8.	Define ‘magnetization’.

9.	State Faraday’s law.

10.	Define ‘skin depth’.

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the relation between cylindrical coordinates of a point and the cartesian coordinates of the same point.

12.	Write the significance of the principle of Superposition with respect to the electro static field.

13.	Define torque and write down the expression for the torque experienced by a current carrying loop of area ‘A’ square meter  situated in a magnetic field of B Wb/m2 .

14.	Write the boundary conditions of magnetic fields.

15.	Explain the concept of displacement current.

PART – C  (5 x 15 = 75 MARKS)

16.	A vector in Cartesian coordinate system is given by A=4ax+3ay+2az. Obtain the vector in the cylindrical coordinate system and spherical coordinate system.

(OR)

17.	Obtain the conversion equations which convert a point in rectangular co-ordinate system into cylindrical coordinate system and spherical co-ordinate system.

18.	Derive an equation to find the electric field intensity at any point along the axis of a uniformly charged circular disc.

(OR)

19.	Three point charges q1 = 10- 6 C, q2 = -10 -6 C and q3 = 0.5 x 10 -6 C are located in the corners of an equilateral triangle of 50 cm side. Determine the magnitude and direction of the force on q3.

20.	Derive an expression to find H  at any point due to an infinitely long straight conductor.

(OR)

[P.T.O]

21.	a.	A wire carrying a current of 50 A is bent into the form of a circle of diameter 100cm. 	Calculate the flux density at the centre of the coil. Assume the number of turns to be 1. (7)

b.	State Ampere’s circuital law and explain its applications.							    (8)

22.	Derive an expression to find the capacitance between two co-axial cables.

(OR)

23.	a.	Determine the inductance of a solenoid of 2000 turns wound uniformly over a length of 10 	cm on a cylindrical paper tube of 4 cm diameter. The medium is air. 				    (8)

b.	Explain about energy and energy density in an electrostatic field.					    (7)

24.	State and prove pointing theorem.

(OR)

25.	Derive wave equation for uniform plane waves in free space and in a good conducting medium.