Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	SOLID STATE CIRCUITS							      	     Time : 3 hours

Subject Code:	12EC207						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The efficiency of  the half wave rectifier is __________.

a.	20.4%   b.	30.7%  	c.	40.6%  	d.	81%

2.	Zener diode maintains a constant output voltage in its ___________ region.

3.	What is meant by stability factor?

4.	Mention the operating region in which the transistor acts as amplifier.

5.	If G1, G2 and G3 are the individual voltage gains of a three stage amplifier, then total voltage gain G = ________.

6.	Which power amplifier eliminates crossover distortion?

7.	What is the major drawback of negative feedback amplifier?

8.	Write the expression for closed loop gain(Af) in positive feedback amplifier.

9.	Name a typical LC oscillator.

10.	What is tuned amplifier?

PART – B  (5 x 3 = 15 MARKS)

11.	What is rectification efficiency?

12.	Consider the fixed bias BJT circuit having VCC =12V, IB =40 µA, VCE = 6V, β=80, VBE  =0.7 V.  Determine a) Collector  current  b) Collector Resistance

13.	Write the functions of the following capacitors in Single Stage Amplifier.

a. Base Capacitor  	b. Emitter Capacitor  	c. Collector Capacitor

14.	Define Common Mode Rejection Ratio(CMRR).

15.	State Barkhausen criterion.

PART – C  (5 x 15 = 75 MARKS)

16.	With a circuit diagram and necessary waveforms describe the operation of a full wave rectifier and derive the expression for dc current, rms current and regulation.

(OR)

17.	Draw a neat circuit diagram of short circuit or over load protection and explain its working.

18.	Draw the fixed biasing circuit of BJT and derive its stability factor.

(OR)

19.	Describe in detail about self-biasing of FET.

20.	Draw the circuit diagram of RC coupled amplifier and explain its working with frequency response.

(OR)

21.	Prove that the maximum efficiency of a series fed, directly coupled class A amplifier is just 25%.

[P.T.O]

22.	Draw the circuit diagram of current shunt feedback amplifier and derive its input impedance, output impedance and voltage gain.

(OR)

23.	Draw the circuit diagram of differential amplifier using BJT and explain its operation.

24.	Explain the operation of Hartley oscillator with neat diagram and derive its frequency of oscillation.

(OR)

25.	With a neat sketch, describe the operation of single tuned amplifier.