Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS													      	     								     Time : 3 hours

Subject Code:	12EC208								 		      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Draw the pin configuration of IC741.

2. 	What is the gain of an inverting amplifier where RF=10KΩ andRi=5KΩ?

3. 	What is a voltage regulator?

4. 	What is the other name for regenerative comparator?

5. 	What is a filter?

6. 	What are the applications of 555 timer?

7. 	Define capture range.

8. 	Mention any one direct type ADC.

9. 	What is thermal oxidation?

10. 	What is metallization?

PART – B  (5 x 3 = 15 MARKS)

11.  What are the ideal characteristics of op-amp?

12. 	State Barkhausen criterion.

13. 	Draw the frequency response of high pass filter.

14. 	Describe the operation of weighted resistor DAC.

15. 	What are the advantages of ICs?

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive the voltage gain expression of inverting and non-inverting amplifier.

(OR)

17.  With a neat sketch, explain about the log amplifier and derive its output voltage V0 .

18. 	Discuss the working of RC phase shift oscillator with a neat circuit and derive its frequency of 	oscillation.

(OR)

19. With a neat circuit diagram, explain about IC723 general purpose voltage regulator.

20. Derive the transfer function of first order low pass filter and draw its frequency response.

(OR)

21. Draw the functional diagram of IC 555 timer and explain its operation.

22. With a neat block diagram, explain the operation of a phase locked loop.

(OR)

23. Explain the working principle of successive approximation type ADC.

24. Explain the steps involved in fabrication of ICs in planar process.

(OR)

25. 	With a neat sketch, explain the fabrication of integrated resistors.