Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	TRANSMISSION LINES AND WAVE GUIDES		      	     Time : 3 hours

Subject Code:	12EC211								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the types of transmission line?

2.	Define waveform distortion.

3.	Why open wire lines become unstable for frequencies above 100MHz?

4.	What are standing waves?

5.	Give some impedance matching techniques used in transmission lines.

6.	What are the applications of smith chart?

7.	Give the cut off wavelength of transverse electromagnetic waves.

8.	Define Attenuation in waveguides.

9.	Give the dominant mode in a rectangular waveguide.

10.	Define wave impedance.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain about loading and its types.

12.	Calculate the standing wave ratio if the transmission line with a characteristic impedance of 50Ω is terminated with an impedance of 75Ω.

13.	Give the significance of circle diagram.

14.	What are the characteristics of TEM waves?

15.	Why TEM wave does not exist in a rectangular waveguide?

PART – C  (5 x 15 = 75 MARKS)

16.	Sketch the equivalent circuit of the transmission line and hence solve its general solution equations.

(OR)

17.	a.	Derive Campbell’s equation of transmission line for loading.						  (10)

b.	Find out the general current and voltage equations of a short circuited line.			   (5)

18.	a.	Mention the assumptions to be made for a transmission line at radio frequencies and 	discuss the parameters of coaxial lines.											  (10)

b.	Derive the primary constants of dissipation less line.								    (5)

(OR)

19.	Discuss about the standing wave pattern and derive the relationship between reflection coefficient and Standing wave ratio. Also discuss about reflection loss.

20.	What is the need of impedance matching in a transmission line and hence derive the input impedance of a half wave and quarter wave line.

(OR)

[P.T.O]

21.	A 50 Ω lossless line is terminated in a load impedance of (25+j50) Ω. Use the smith chart to find the following, given the length of the line is 3.3λ long

a.	Locate the load impedance and admittance. 										    (2)

b.	VSWR circle. 																    (2)

c.	Vmax and Vmin. 																    (2)

d.	Normalized input impedance and input admittance. 								    (2)

e.	Input Impedance and admittance. 												    (2)

f.	Magnitude and angle of Voltage reflection coefficient at source and load. 			    (2)

g.	Draw the length of the line from source to load along the wavelength scale. 			    (3)

22.	Derive the electric and magnetic components of transverse electric waves in a parallel plate waveguide.

(OR)

23.	A pair of perfectly conducting planes is separated by 8 cm in air. For frequency of 5000 MHz with the TM11 mode excited, find the following:

a.	Cut off frequency.

b.	Wave impedance.

c.	Attenuation constant if f = 0.9 fc.

d.	Phase constant and phase velocity.

24.	Discuss the application of Maxwell’s equation to TM waves of Rectangular waveguides.

(OR)

25.	Briefly discuss about the following resonators with neat diagrams:

a.	Rectangular cavity Resonator. 													    (8)

b.	Circular cavity Resonator. 														    (7)