Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	SIGNALS AND SYSTEMS 								      Time : 3 hours

Subject Code:	12EC214								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define random signal with an example.

2.	The system y(t)=ex(t) is a static system. (True/False)

3.	Fourier transform of (t) is ___________.

4.	What is meant by Region of Convergence (ROC)?

5.	In natural sampling, sampling signal used is ___________.

a. train of impulses                b. train of pulses

c. unit step signal                   d. exponential signal

6.	What is aliasing?

7.	F[x(–n)]= ___________.

8.	Z-transform of x(n)=u(n)–u(n-1) is ___________.

9.	Define impulse response.

10.	MATLAB represents ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	List the different classification of system.

12.	Mention the conditions required for the existence of Fourier transform.

13.	State Sampling theorem.

14.	Find the discrete time Fourier transform of u(n–k).

15.	Differentiate FIR and IIR system.

PART – C  (5 x 15 = 75 MARKS)

16.	Represent the basic elementary continuous time signals mathematically and graphically. Also, explain the different types of operations on signals with example.

(OR)

17.	Define convolution sum and list its different properties. Find the output of the system using convolution for the given input x(n)=14nu(n)and impulse response h(n)=u(n)-u(n-7).

18.	a.	Determine the Fourier transform and sketch the magnitude and phase spectrum for the 	following.

i.	x(t) = e-5t u(t)

ii.	x(t) = e-3|t|																    (9)

b.	Prove Parseval’s theorem and convolution property in continuous time Fourier transform.

(6)

(OR)

19.	Realize the system described by differential equation in direct form-I and direct form-II.

d3y(t)dt3+5d2y(t)dt2+6dy(t)dt+9y(t)=dx(t)dt+5x(t)

[P.T.O]

20.	Mention different sampling methods. Explain impulse train sampling and reconstruction of continuous time signal from its samples.

(OR)

21.	The signal x(t)=10cos(10πt) is sampled at a rate 8 samples per second. Plot the amplitude spectrum ||≤30π. Can the original signal can be recovered from samples? Explain.

22.	State and prove any five properties of Discrete Time Fourier transform.

(OR)

23.	a.	Find the Z-transform and ROC of the following signals.							    (8)

i.	x(n)= 12nu(−n)+14nu(n)

ii.	x(n)= 12nu(n)+13nu(n)

b.	Find the inverse Z-transform of X(z)=14z−1(1−12z−1)(1−14z−1). ROC:|z|>12			    (7)

24.	Draw Direct Form I and Direct Form II implementations for the difference equation

y(n)+12y(n−1)−18y(n−2)=x(n)+2x(n−1).

(OR)

25.	Find the impulse response, frequency response and step response for the given difference equation y(n)−34y(n−1)+18y(n−2)=x(n).