Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MICROPROCESSORS AND INTERFACING TECHNIQUES													      	     								     Time : 3 hours

Subject Code:	12EC215								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.     What is meant by instruction cucle?

2.     The processor has a __________ register that holds the memory address of the next instruction.

3.     Give example for two byte instruction in 8085.

4.     8085 follows _________ architecture.

5.	The width of the data bus of 8086 is __________.

6.	The size of instruction queue in 8086 is __________.

7.	Serial communication interface data transfer rate is called as __________.

8	Define USART.

9.	The IC used to interface printer and control it from the processor is __________.

10.	What are the two keyboard entry modes?

PART – B  (5 x 3 = 15 MARKS)

11.	List out the registers used in basic computers.

12.  Write a program to complement a number in 8085.

13.  How is the 20-bit effective address calculated in an 8086 processor?

14.  Name two 16 bit registers used in DMA controller.

15.	Difference between timer and counter.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With a neat diagram, Explain the design of Accumulator logic. 					    (7)

b.	List out the memory and register reference instructions available in basic computers.    (8)

(OR)

17.	Explain the control unit operation in basic computer.

18.  Draw and explain the architecture of 8085 microprocessor.

(OR)

19.	a.	Write an assembly language program to multiply two numbers in 8085.				    (8)

b.	With a neat timing diagram, Mention how many machine cycles are needed to execute the following instruction, 4300: MVI A, 03H.  										    (7)

20. 	a.	Explain minimum and maximum mode operation in 8086.							  (10)

b.	Draw the archiecture of 8086 microprocessor.									   (5)

(OR)

21.	a.	Write a program in 8086 used to transfer a block of data from one location to other.     (7)

b.	Explain in detail about Memory Segmentation in 8086 microprocessor. 				    (8)

[P.T.O]

22.	Draw and explain how 8255 can be interfaced to 8085 with the explanation of all the signals involved.

(OR)

23.	What is the use of DMA? Explain with a neat diagram, how it is interfaced with microprocessor.

24.	When do you need a programmable interval timer? Explain the features, internal block diagram and operating modes of 8253 Timer.

(OR)

25. What is the significance of using 8279? Explain the operation with neat block diagram.