. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	12EC216						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In which method the overlapped samples in the output sequences are added?

2.	What is zero padding?

3.	Give any one characteristic of a chebyshev filter.

4.	Relate s-plane and z-plane using bilinear transformation.

5.	What are the advantages of an FIR filter?

6.	Write down the Hamming window function.

7.	Find the 2’s complement of (1001)2.

8.	Define quantization step size.

9.	What is the need of adaptive filtering?

10.	Differentiate general purpose and special purpose processors.

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the linear convolution of the following sequences.

x(n) = {1,2,1}; h(n) = {1,-2,2}.

12.	Design an analog high pass filter with pass band attenuation of 3 dB at 1000 rad/sample and stop band attenuation of 15dB at 500 rad/sample.

13.	Differentiate IIR and FIR filters.

14.	Define truncation and rounding with an example.

15.	What are the limitations of wiener filter?

PART – C  (5 x 15 = 75 MARKS)

16.	Using the radix-2 decimation-in-time FFT algorithm, compute the 8 point DFT of the sequence. Draw the flow graph and show all the intermediate results.

x(n) = {0.5, 0.5, 0.5, 0.5, 0, 0, 0, 0}

(OR)

17.	Perform the linear convolution of the following sequence by overlap save method:

x(n) = {3,-1,0,1,3,2,0,1,2,1} and

h(n) = {1,1,1}

18.	a.	Design a Chebyshev filter for the following specifications using Bilinear transformation technique. Assume T = 1 sec.

0.707≤H(ejω)≤1 for 0≤ ω ≤ 0.2π  H(ejω)≤0.1 for 0.5π≤ ω ≤ π

(OR)

19.	Design a digital Butterworth low pass filter using impulse invariance method whose transfer function is given by

Pass band attenuation : 0.7

Pass band edge frequency	:0.2 rad

Stop band attenuation : 0.3

Stop band edge frequency : 0.6 rad

[P.T.O]

20.	Design an ideal high pass filter with the following frequency response using a Hanning window. Find the values of h(n)for

N=11. FindH(z). Plot the magnitude response.

Hd(ejω) =1 for π4≤ |ω| ≤ π0 for |ω| ≤ π4

(OR)

21.	Using frequency sampling, Determine the coefficients of a linear phase FIR filter of length N=15 which has a symmetric unit sample response and a frequency response that satisfies the conditions

H2πk15=  1 for k=0,1,2,30 for k=4,5,6,7

22.	a.	Explain coefficient quantization error with an example.							    (7)

b.	Find the steady state variance of the noise in the output due to quantization of input for the first order filter y(n)=ay(n−1)+x(n).											    (8)

(OR)

23.	Explain signal scaling to prevent overflow limit cycle in the second order digital filter implementation.

24.	Draw and explain the architecture of TMS320C5416.

(OR)

25.	a.	Explain the least mean square (LMS) adaptive algorithm in detail.					    (8)

b.	Explain the process of pipelining in detail.										    (7)