Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MODERN CONTROL SYSTEMS					      	     Time : 3 hours

Subject Code:	12EC220						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a signal flow graph, a node with all incoming edges is known as __________.

2.	Write Mason’s Gain formula.

3.	Define steady state error.

4.	Define delay time.

5.	Mention any two frequency domain specifications.

6.	In Nichols chart the M contours are __________ and N contours are __________.

7.	If the characteristic equation has repeated roots on jw axis, the system is ___________.

8.	“The necessary and sufficient condition for stability is that all of the elements in the first column of the Routh array be __________”.

9.	Define state space.

10.	Write down the general representation of state equation and output equation.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare Open Loop and Closed Loop system and give example for each.

12.	How the system is classified depending on the value of damping ratio?

13.	What is gain and phase cross over frequency?

14.	What is centroid? How the centroid is calculated?

15.	Write the characteristic equation for the given matrix.

A=14−2−5

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the equivalent signal flow graph for the given block diagram and obtain the transfer function. Verify the same using block diagram reduction techniques.

+

+

+

-

+

-

+

-

R(s)

G1

G2

G4

G3

H1

H2

C(s)

[P.T.O]

(OR)

J1

J2

K

T    (Torque)

 (Output)

17.	Obtain the transfer function of the mechanical rotational system shown in the figure

18.	Obtain the time response of second order under damped system when subjected to unit step input signal.

(OR)

19.	A unity feedback control system has an open loop transfer function GS=10SS+2. Find the rise time, Percentage overshoot, Peak time and settling time for a step input.

20.	Draw the Bode plot and find gain margin and phase margin for the following open-loop system.

GS=S1+0.2S1+0.02S.

(OR)

21.	The open loop transfer function of a unity feedback system is given by

GS=1ss+12s+1

Sketch the polar plot and determine the gain margin and phase margin.

22.	Obtain the root locus for the following open-loop transfer function GS=Kss+1s+2.

(OR)

23.	a.	Apply Routh  criterion to check the stability of the following characteristic equation

S6+9S5+20S4+12S3+8S2+16S+16=0.											(5)	b.	A unit-feedback system is characterized by the open-loop transfer function

GS=KSS+3S+7

Using the Routh criterion, calculate the range of values of K for the system to be stable.

(10)

24.	Check Controllability and Observability for the following

x1x2=−2101x1x2+11u

y=11x1x2

(OR)

25.	Determine Eigen values and Eigen Vectors

A=3421