Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DIGITAL DESIGN USING VHDL					      	     Time : 3 hours

Subject Code:	12EC232						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In PLA AND array are _________ and OR array are_________.

2.	Name the different programming technologies.

3.	Give the abbreviation for ABEL.

4.	What are the sequences of instructions in micro programmed controller?

5.	What are the two types in oxidation?

6.	Define full custom design.

7.	Concurrent signal assignment statements are executed whenever _________ occur on signals.

8.	Give the syntax of IF statement.

9.	Define entity.

10.	_________ delay is called default delay.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate PROM and PAL.

12.	What are the advantages and disadvantages of Micro programmed control unit?

13.	Discuss about standard cell design.

14.	Write dataflow model VHDL program for full adder circuit.

15.	By using CASE statement write VHDL code for a 4X1 multiplexer.

PART – C  (5 x 15 = 75 MARKS)

16.	Implement the functions using PLA

A(x,y,z)= Σm(1,2,4,6)

B(x,y,z)= Σm(0,1,6,7)

C (x,y,z) = Σm(2,6)

(OR)

17.	a.	Implement the following Boolean logic equations in PLA.

W=Q2Y+Q3Y

A1=YQ2Q3

A2=Q2Q3+YQ1+YQ2Q3

A3=Q2Y+Q3Y																    (5)

b.	Implement the binary to gray code converter (3 bits) using PROM where A, B, C are 	inputs and W, X, Yare outputs.      												  (10)

18.	a.	Design a Micro programmed Controller and prepares micro instructions to perform ADD 	macro. Justify your use of instructions.											  (10)

b.	Give some lists of micro-instruction.											    (5)

(OR)

[P.T.O]

19.	a.	With the design flow, lists the different operators and sub-programs used in ABEL.	  (10)

b.	 Discuss about emulation.														    (5)

20.	Explain in detail IC Design flow process from specification to packaging and testing.

(OR)

21.	With neat diagram explain in detail about Xilinx 3000 FPGA and also describe how a 32-bit LUT can realize different functions.

22.	a.	Write a VHDL Program for four bit full adder, using the structural description. Draw its 	basic structure showing the inputs and outputs signals.							  (10)

b.	List the types of data operators available in VHDL with examples.					    (5)

(OR)

23.	Explain in detail about different scalar data types.

24.	a.	Design a 2 bit magnitude comparator in Behavioral modeling using VHDL (use if-then-	else OR CASE statement).													  (10)

b.	Write VHDL program for half adder using structural modeling.					    (5)

(OR)

25.	Explain in detail about selected signal assignment statement in VHDL with an example.