Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	VIRTUAL INSTRUMENTATION					      	     Time : 3 hours

Subject Code:	12EE237/EE270					 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	LabVIEW is Platform Independent – True/False.

2.	VI can be implemented using graphical and __________ programming languages.

3.	A Block diagram node executes when all __________ are available.

4.    __________ is the maximum no of terminals/objects that can be used to create a Sub VI.

5.	The two types of sequence structures available in LabVIEW are __________ and __________.

6.	For Loop executes a subdiagram for __________ of times in LabVIEW.

7.	Signals can be classified into two groups namely __________ and __________.

8.	NRSE Stands for __________.

9.	Measurement and Automation explorer allows you to configure the __________.

10.	Serial Data communication is achieved using __________ in LabVIEW.

PART – B  (5 x 3 = 15 MARKS)

11.	List a few advantages of VI.

12.	List the data Types used in LabVIEW.

13.	When are Local and Global variables used?

14.	List the steps to create a Task for measurement of Temperature using Thermocouple.

15.	What are three terminology which are common to VISA and instrument driver VI’s?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	List the difference between Traditional and Virtual Instrumentation	.				    (5)

b.	With a necessary diagram explain the architecture of a virtual Instrument.			  (10)

(OR)

17.  What is LabVIEW? How graphical programming is different from text based 	programming? 
Explain data flow programming concept with help of a VI.

18.	a.	List and explain the advantages of LabVIEW.									    (7)

b.	Write in detail about the software environment.									    (8)

(OR)

19.	What is the purpose of Icons and Connectors? Consider an example and explain how to create a 
Sub VI.

20.	What are arrays and clusters? Draw and explain with examples the following functions

a.	Cluster Order.

b.	Error cluster.

c.	Bundle/unbundle Cluster.

d.	Bundle/ Unbundle by name.

(OR)

[P.T.O]

21.	a. 	Create a VI to convert decimal numbers to binary numbers using for loop.			  (10)

b. 	What is a tunnel how is it used in loops?											   (5)

22.	Elaborate on the DAQ hardware with block diagram.

(OR)

23.	a. 	List the steps to configuration the DAQ hardware. 								  (10)

b. 	Explain about Signal Conditioning in LabVIEW data acquisition.					    (5)

24.	Explain with a required block diagram about VXI and PXI.

(OR)

25.	With required diagrams explain GPIB communication using LabVIEW.