Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	HIGH VOLTAGE ENGINEERING					      	     Time : 3 hours

Subject Code:	12EE257/EE271									       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The object of good line design is to ________ the number of outages caused by lightning.

2.	How does switching over voltage originate?

3.	Analyze the metastable interaction.

4.	What is parallel plate capacitor?

5.	Write the expression to find the optimum number of stages and %ripple in a voltage multiplier circuit.

6.	Distinguish between electromagnetic and electrostatic machines.

7.	Sphere gap is used for measurement of ________.

8.	Give any two advantage of electrostatic voltmeter.

9.	What is bending test in cables?

10.	Write any two tests for testing bushings.

PART – B  (5 x 3 = 15 MARKS)

11.	Define tower footing resistance.

12. 	All the photons are electromagnetic waves. Justify.

13.	Analyze the necessity of generating High DC voltages.

14.	Express the output voltage of voltage diverter.

15.	List out various tests to be carried out on insulator.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Describe the variation of current and voltage on an overhead line when one end of the line 	is (i) short-circuited, and (ii) open-circuited and at the other end a source of constant e.m.f. 	V is switched in to the transmission line.											  (10)

b.	A surge of 15 kV magnitude travels along a cable towards its junction with an overhead line. The inductance and capacitance of the cable and overhead line are respectively 0.3 mH, 0.4 μF and 1.5 mH, 0.012 μF per km. Find the voltage rise at the junction due to the surge.																		    (5)

(OR)

17.	a.	Define ground wire. Discuss its location with respect to power conductors. 			  (10)

b.	Express the transient current waveform in RC circuit.			  					    (5)

18.	Discus various factors which affect breakdown of solids.

(OR)

19. 	Explain the mechanism of development of anode and cathode steamers and explain how these lead to breakdown.

20. 	Describe with neat diagram the principle of operation, advantages, limitations and applications of Van de Graff generator.

(OR)

[P.T.O]

21. 	Describe the construction, principle of operation and application of a multistage marx’s surge generator.

22. 	Draw a neat schematic diagram of a generator voltmeter and explain its operation and give the application.

(OR)

23. 	What is Rogowski Coil? Explain with a neat diagram its principle of operation for measurement of high impulse currents.

24. 	List out various tests to be carried out on a cable and give a brief account of each test.

(OR)

25. 	Explain briefly various tests to be carried out on a circuit breaker.