Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	12EI211						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is fan in and fan out?

2.	What is TTL logic?

3.    Convert the binary number (101111.1101)2 to its decimal and octal equivalent.

4.	What do you understand by weighted codes? Give example.

5.	Implement OR gate using NAND gates.

6.	What is a combinational logic circuit?

7.	Differentiate multimode counters and modulus counters.

8.	Mention at least two applications of T flip-flop.

9.	What is a FPGA?

10.	PAL consists of __________ .

PART – B  (5 x 3 = 15 MARKS)

11. 	What is totem pole configuration? What is its advantage.

12. 	What are error detecting and error correcting codes?

13. 	Draw the structure of one bit comparator.

14. 	Draw the complete structure of JK flip flop and write its excitation table.

15. 	Differentiate the basic models of synchronous sequential machine.

PART – C  (5 x 15 = 75 MARKS)

16.	With neat diagram explain the working of CMOS NAND and NOR gates.

(OR)

17.	Briefly describe the various parameters of logic families. Explain the working of TTL logic as 3 input NAND gate.

18.	a.   Write the canonical form of the expression

Y(ABC)=(A+B)(B+C)(A+C)												    (5)

b.	Simplify the Boolean function using K-map and implement using NOR-NOR logic

Y(ABCD)= ∑(0,1,2,4,5,6,8,9,12,13,14)											  (10)

(OR)

19.	Use Quine Mckluskey method to simplify and implement using NAND –NAND logic.

f(ABCD)= ∑(0,2,3,6,7,8,10,12,13)

20.	a.	Design and implement a 4-bit  gray to binary code converter.						  (10)

b.	Design a priority encoder.														    (5)

(OR)

21.	a.	Discuss the design and implementation of 4 bit adder/subtractor circuit.				    (8)

b.	Draw the structure of a 4*1 multiplexer.											    (7)

[P.T.O]

22.	Explain the operation of 4 bit Johnson counter and ring counter using timing waveforms.

(OR)

23.	Design a Mod-20 ripple counter. Use JK flip flops. What is the disadvantage of ripple counter?

24.	a.	Define the different types of PLD’s.    											    (5)

b.	Design a PLA to implement the function. 										  (10)

Y0= AB+A’CB’

Y1=AB’C+AB+AC’

Y2=AB+BC+CA

(OR)

25. 	What is a HDL? Explain its significance. Explain the different types of Modeling in VHDL with Multiplexer as an example.